Space Agency Response to GCOS Implementation Plan

Space Agency Response to GCOS Implementation Plan

© The Joint CEOS/CGMS Working Group on Climate (WGClimate), 2017.

Document Reference ESA-ECO-EOPS-WGCL-RP-17-0061. Version 1.0

The right of publication in print, electronic and any other form and in any language is reserved by WMO. Short

extracts from WMO publications may be reproduced without authorization, provided that the complete source

is clearly indicated.

Table of Contents

Executive Summary	vi

1	Introduction	1

1.1	Background	1

1.2	Purpose of this document	1

1.3	Approach to Responding to GCOS Implementation Plan	1

1.4	Terminology	2

2	The Architecture for Climate Monitoring from Space	4

2.1	Role of Satellites in a Global Climate Observing System	4

2.2	The Joint CEOS/CGMS Working Group on Climate	5

3	Response to Part I: Broad Context	7

3.1	Observations for Adaptation, Mitigation and Climate Services	7

3.2	The Broader Relevance of Climate Observations	9

3.3	Consistent Observations Across the Earth System Cycles	9

3.4	Capacity Development and Regional and National Support	10

3.5	Calibration and Validation of Satellite Data, Assessment and Intercomparison	11

4	Response to Part II : Detailed Implementation	12

4.1	Overarching and Cross-Cutting Actions	12

4.2	Atmospheric Climate Observing System	16

4.3	Oceanic Climate Observing System	23

4.4	Terrestrial Climate Observing System	31

Annex A.	Acronyms	32

Annex B.	Mapping From GCOS IP to Space Agency Response	37

Annex C.	Contributors	44

# Executive Summary

The Committee on Earth Observation Satellites (CEOS) and Coordination Group for Meteorological Satellites (CGMS), in the form of the Joint CEOS/CGMS Working Group on Climate (WGClimate), are pleased to present a response to the 2016 GCOS Implementation Plan, reiterating their commitment to address the Actions required for the implementation of the Global Climate Observing  System (GCOS). CEOS and CGMS, international organizations of 60 Members and Associates and 15 Members respectively, have had the honour to report on Space Agency activities to GCOS on several previous occasions.

Space Agencies have continued to evolve their systematic observation of the climate system, now spanning several decades, strengthening scientific knowledge on climate, and supporting the provision of knowledge-based information to climate services to under-pin informed decision-making. Space Agencies are framing their activities through the implementation of the Strategy Towards an Architecture for Climate Monitoring from Space, 2013 – developed by a team comprised of representatives from CEOS, CGMS, and the World Meteorological Organization (WMO).

The Role of the Architecture for Climate Monitoring from Space

In response to the continuing challenge of monitoring climate variability and climate change from space, CEOS, CGMS and WMO have accelerated development of the global Architecture for Climate Monitoring from Space. A recent, substantial, advance in the development of the ECV Inventory has led to an unprecedented insight into the activities of Space agencies across the ECV spectrum, resulting in the detailed characterisation of over 900 Climate Data Records (CDRs) that directly respond to the GCOS ECV requirements. The ECV Inventory enables the identification of existing and potential future gaps in the provision of the climate data requested by GCOS.

An analysis of the gaps between GCOS needs and the CDRs delivered by Space Agencies is currently underway, and will include the identification of actions to close any identified gaps. We are pleased to leverage this activity for the direct benefit of GCOS, thereby more accurately targeting, and responding to, GCOS needs.

The Architecture for Climate Monitoring from Space is central to our response to the GCOS Implementation Plan, and frames our response to both the Broad Context and Detailed Implementation actions articulated by GCOS.

Broad Context

Space Agencies are implementing programmes explicitly aimed at producing CDRs that respond to the ECVs requested by GCOS. Moreover, climate services directly benefit from the wealth of space-derived climate data products being systematically generated by Space Agency programmes. The continuing implementation of the global Architecture for Climate Monitoring from Space, and in particular the ECV Inventory, provides strong evidence of the on-going commitment of Space Agencies in terms of the application of sustained resources and investments.

Efforts have accelerated at a regional level also. For example, the WMO designated Regional Climate Centres (RCCs) are being implemented to generate and deliver more regionally-focused high-resolution data and products, as well as training and capacity building. The WMO has been making concerted efforts to implement RCCs, in close coordination with its Regional Associations, Commission for Climatology and Commission for Basic Systems.

On mitigation, CEOS continues to support the Global Forest Observations Initiative implementing the United Nations Collaborative Programme on Reducing Emissions from Deforestation and Forest Degradation or REDD+.

Detailed Implementation including Preliminary Identification of Gaps

The overall response by Space Agencies to Atmosphere-related Actions is strong, although some important gaps remain. The meteorological satellite constellation, coordinated by CGMS, provides strong heritage and a backbone for satellite data characterising atmospheric physics, dynamics, and composition. Many agencies have launched programmes for generating satellite CDRs, and international coordination through Sustained Coordinated Processing of Environmental Satellite Records for Climate Monitoring (SCOPE-CM) has improved efficiency. Space Agencies are also enhancing coordination of a satellite constellation for monitoring CO2 and CH4, and are stepping up their investments. Unresolved gaps include the uncertain future of satellite-based measurements of precipitation, and no plans for continuing measurements of the upper tropospheric and stratospheric profile for better understanding the Earth's radiation balance, among other benefits. In addition, a space-based calibration mission could not be realized in full, and shortfalls have been identified in following up on key research missions that have revolutionised our understanding of atmospheric processes (e.g., aerosol-cloud interaction by measuring the 3D structure of clouds and aerosol layers).

Our uncertainty of Terrestrial ECV products is a function of the temporal resolution of satellite data acquisition and the availability of in situ observations.  Due to cloud cover, and low illumination conditions at high latitudes, significantly higher errors occur when optical data are used in isolation. Uncertainty is also difficult to assess, as in situ data are extremely limited and, in general, not consistent in space and time. Satellite multi-sensor and multi-product inter-comparisons have been minimal so far for lake temperature, as the availability of a reasonable set of overlapping products is a pre-requisite for such activities.  Lake in situ monitoring and satellite observation targets need to be extended, and a common database derived on which to focus production and subsequent validation activities for all products.  A higher spatial resolution is required for monitoring lake ice extent. Temporal resolution has considerably improved for many Arctic regions as more daily data are acquired. There is a strong need to establish lake sites for algorithm development and validation, for the purpose of long-term monitoring.

A major drawback for the true monitoring of constantly changing glaciers in steep high-mountain topography is the missing availability of Digital Elevation Maps (DEMs) from high-resolution sensors for ortho-rectification of the related satellite data. For Glacier area the requirement for annual frequency is unnecessary because decadal change is sufficient.  For glacier elevation change this is opportunistic and depends on DEM availability, quality and characteristics. For glacier flow velocity, observations on a sub-annual basis are possible using SAR and optical sensors especially for surge events, other instabilities, and seasonal velocity variations.  Existing Fire ECV products are approaching target requirements for spatial resolution, although may not reach temporal requirements, but merged products derived from multi-sensor data may be close to satisfying both requirements.  The International Land Surface Temperature and Emissivity Working Group (ILSTE, ilste-wg.org) has been initiated to bring developers and producers together to improve coordination and to make products more accessible to users.

Space Agencies intend a strong response overall to actions related to Oceans, with some gaps. Meteorological and operational optical and thermal missions will provide a strong instrumental backbone for sea surface temperature, with areas requiring stronger action focussed around maximising the quality of long records and integrating diurnal cycle observations from geostationary platforms. For SST and sea ice, the absence of plans across many agencies for low-frequency passive microwave radiometry remains concerning. An increased emphasis on evolving better ocean heat flux and sea surface dynamic state products constrained by observations needs to be sustained. Co-operation and continuity in sea surface height / sea level are strong, although validation and uncertainty estimation remain challenging. Observational continuity for ocean surface reflectance observation for ocean biogeochemistry looks secure from polar orbiting platforms, and effort is strong in radiometric validation. Further product evolution is needed for phytoplankton biomass estimation and long-term climate data record stability. The global salinity record from L-band microwave frequencies is progressing, with multi-mission synthesis a priority. Salinity mission extensions are planned or under review that will extend the record beyond a decade, although no new salinity missions are in the pipeline thereafter.

As Agencies continue to invest in the efforts necessary to create CDRs from their archives of observations it is also understood that CDRs must have stringent quality characteristics that enable quantitative analysis of climate change and variability over decades. Therefore Space Agencies are addressing in situ fiducial reference measurements for verifying the accuracy and stability of satellite CDRs, integration of CDRs across evolving constellations of sensors for a particular ECV, assessment of consistency between variables, and reducing the knowledge gaps in the understanding of uncertainty in CDR products across all spatial/temporal scales relevant to climate. These activities have been applied across the atmospheric, oceanic and terrestrial domains, and substantial progress has been made for a significant number of CDRs.

# Introduction

## Background

The Global Climate Observing System (GCOS), a joint undertaking of the World Meteorological Organization (WMO), the Intergovernmental Oceanographic Commission (IOC) of the United Nations Educational, Scientific, and Cultural Organization (UNESCO), the United Nations Environment Programme (UNEP) and the International Council for Science (ICSU), was established in 1992 to ensure that the observations and information needed to address climate related issues are obtained and made available to all potential users.

At the 7th Conference of the Parties (COP 7) to the United Nations Framework Convention on Climate Change (UNFCCC) in 2001, the UNFCCC Subsidiary Body on Scientific and Technological Advice (SBSTA) invited GCOS to consider an integrated (satellite and in situ) approach, including the exploitation of new and emerging methods of observation to the measurement of climate change. At COP 9 in 2003, GCOS was invited to develop a phased 5-10 year implementation plan. COP 10 in 2004 invited Parties with space agencies to have those space agencies provide a coordinated response to the recommendations in the 2004 implementation plan.

## Purpose of this document

The purpose of this report is to provide a consolidated Space Agency response to actions from the Global Climate Observing System in the form of GCOS 200, recently updated from GCOS 154. Technical supplements responding to actions per ECV in detail will be provided in early 2018.

## Approach to Responding to GCOS Implementation Plan

This document comprises the following parts :

- Introduction (§1).

- The Architecture for Climate Monitoring from Space (§2).

- Response to Part I : Broad Context (§3).

- Response to Part II : Detailed Implementation (§4).

- Acronyms (Annex A).

- Mapping from GCOS IP to Space Agency Response (Annex B),

- Contributors (Annex C).

## Terminology

Essential Climate Variable

An Essential Climate Variable (ECV) is a geophysical variable that is associated with climate variation and change as well as the impact of climate change onto Earth. GCOS has defined a set of ECVs for three spheres, atmospheric, terrestrial and oceanic,.

ECV Product

Many GCOS ECVs are sub-divided into so called ECV Products. The term “Product” denotes long-term data records of values or fields of ECVs derived from FCDRs. For instance the water vapour ECV has 3 ECV Products: total column water vapour, tropospheric and lower-stratospheric profiles of water vapour, and upper tropospheric humidity, which describe different aspects of water vapour in the atmosphere. However, the definitions provided for ECV6 are not always consistent for all ECVs. For some cases, e.g. the ECV Sea Ice, there is only one ECV Product that covers 4 different variables/parameters for which requirements are provided. The ECV Inventory and the associated questionnaire names each variable/parameter for which requirements are provided an ECV Product. A consistent mapping is provided in the ECV Inventory Questionnaire Guide.

GCOS Requirements

A GCOS Requirement as used in this document refers to the quantitative requirements provided for each ECV6 They denote Horizontal Resolution, Vertical Resolution, Temporal Resolution, Accuracy and Stability.

Fundamental Climate Data Record

A Fundamental Climate Data Record (FCDR) is a well-characterised, long-term data record, usually involving a series of instruments, with potentially changing measurement approaches, but with overlaps and calibrations sufficient to allow the generation of products that are accurate and stable, in both space and time, to support climate applications. FCDRs are typically calibrated radiances, backscatter of active instruments, or radio occultation bending angles. FCDRs also include the ancillary data used to calibrate them. The term FCDR has been adopted by GCOS and can be considered as an international consensus definition.

Thematic Climate Data Record

A Thematic Climate Data Record (TCDR) is the counterpart of the FCDR in geophysical8. It is closely connected to the ECVs but strictly covers one geophysical variable, whereas an ECV can encompass several variables. For instance, the ECV Cloud Properties includes at least five different geophysical variables, each of them constituting a TCDR. The term TCDR has been taken up by many space agencies and can be considered as de facto standard.

Interim Climate Data Record

An Interim Climate Data Record (ICDR) is a Fundamental or Thematic Climate Data Record regularly updated with an algorithm / system having maximum consistency to the FCDR or TCDR generation algorithm / system. The update cycle depends on the user needs for climate extremes and might range from pentad to monthly.

# The Architecture for Climate Monitoring from Space

The space-based architecture for climate monitoring forms the major international reference for the contribution of space agencies towards a fulfilment of GCOS goals. It is also reflecting the contribution of space agencies to the monitoring and observation pillar of the Global Framework of Climate Services (GFCS). The implementation of this architecture is coordinated by the joint CEOS/CGMS Working Group Climate (WGClimate) that was established in 2013.

Role of Satellites in a Global Climate Observing System

Earth observation satellites provide a vital means of obtaining observations of the Earth system from a global perspective and comparing the behaviour of different parts of the globe for many of the Essential Climate Variables. Their global nature distinguishes satellite observations from ground-based and airborne measurements that are more limited in spatial coverage, but nevertheless necessary to constrain and validate information derived from space, and provide data on variables not accessible from space.

Satellite climate data records that meet the GCOS requirements enable climate monitoring, studies of trends and variability, climate research, assimilation into numerical weather prediction models to produce long-term reanalyses of Earth System components, provision of boundary conditions for and verification of climate models, climate impact modelling, and, ultimately, decision making in many societal sectors including agriculture, water resource and coastal management, forestry, transportation, and insurance applications. Through this satellite observations play also an important role realizing the potential of Earth observations and geospatial information to advance the GEO 2030 Agenda and enable societal benefits through achievement of the Sustainable Development Goals (SDGs).

In principle, reliable space-based observations can support authoritative statements on climate change by climate services needed to empower international organizations, governments and the private sector to make informed decisions on prevention, mitigation, and adaptation strategies (see Figure 1 for the value adding chain). However, the past and the current observing systems have not been primarily designed with a climate perspective, therefore inventories are needed to document the contributions of current and planned observing systems for climate purposes.

The architecture calls for a constellation of research and operational satellites, broad, open data-sharing policies and contingency planning. It includes agreements that are essential for bringing continuity to long-term and sustained climate observations that were established for weather observations.  The sustained involvement of both research and operational agencies is a prerequisite for success. The Research and Operations paradigm envisioned by the architecture demands the sustained expert understanding of new and legacy sensors and many support activities to derive the needed ECV climate data records. This requires the continued effort of both research and operational agencies which is organized by the CEOS/CGMS Working Group Climate.

Figure 1 : Value adding chain schematically describing the use of satellite-based information for decision making (from Dowel et al., 2013).

## The Joint CEOS/CGMS Working Group on Climate

A Joint Group and Joint Response

The joint CEOS/CGMS Working Group Climate was founded in 2013 to enable all major space agencies to contribute via a coordinated CEOS and CGMS approach to the implementation of the architecture for space based climate monitoring. This joint working group, in addition to facilitating the implementation of the architecture, also provides a coordinated and unique voice for the response of space agencies to the GCOS and GFCS needs and plans.

The over-arching goal of the CEOS/CGMS Working Group on Climate (WGClimate) is to improve the systematic availability of Climate Data Records including the GCOS ECVs through the coordinated implementation, and further development of the architecture for climate monitoring from space. More specifically, the coordination is designed to achieve three main objectives:

- Provision of a structured, comprehensive and accessible view as to what Climate Data Records are currently available from satellite missions of CEOS and CGMS members, or their combination;

- Creation of the conditions for delivering further Climate Data Records, including multi-mission Climate Date Records, through best use of available data to fulfil GCOS requirements (e.g. by identifying and targeting cross-calibration or re-processing gaps/shortfalls );

- Optimisation of the planning of future satellite missions and constellations to expand existing and planned Climate Data Records, both in terms of coverage and record length, and to address possible gaps with respect to GCOS requirements.

The first and third objectives are collectively fulfilled via the establishment of an ECV Inventory and the analysis of its content with respect to the needs of GCOS also addressing the level of compliance with the GCOS Climate Monitoring Principles and the Guidelines for the Generation of Datasets and Products and GCOS requirements as provided in the GCOS Implementation Plan.

The second objective is addressed by multi-partner projects, in particular those implemented in the frame of the WMO SCOPE-CM and GSICS initiatives.

The WGClimate supports the work of GCOS in defining and delivering the ECVs required by the UNFCCC and supports the overall relation of CEOS to the UNFCCC, its subsidiary bodies, and to the Intergovernmental Panel on Climate Change (IPCC).

The WGClimate has also been tasked by CEOS and CGMS to coordinate the response of space agencies to the GCOS implementation plan, and to report to SBSTA/UNFCCC on CEOS/CGMS climate actions.

Moreover, WGClimate and its member agencies are committed to continuing to apply the GCOS Climate Monitoring Principles and guidelines. In addition to the space segment, space agencies resource data management systems that facilitate access, use and interpretation of data and products, treating these as essential elements of climate monitoring systems.

# Response to Part I: Broad Context

## Observations for Adaptation, Mitigation and Climate Services

### Adaptation

Guidance and best practice for adaptation observations (GCOS Action G1)

Space agencies are instrumental in providing capability to meet adaptation needs, by addressing the actions of the GCOS Implementation Plan via the Architecture for Climate Monitoring from Space. These capabilities include, for example, coordinating observation systems to inform adaptation stakeholders, providing public access to high quality and standardized data on the vulnerabilities to climate change impact, and operationalizing the addressing of adaptation at national, regional and local scales.

WGClimate will continue to support, and liaise closely with, GCOS on its continuing adaptation efforts as defined in GCOS-200 (§3.1), and spanning all adaptation action categories, namely requirements and guidance, acquisition of data, data, stewardship, climate services and international cooperation.

Specification of high-resolution data (GCOS Action G2)

WGClimate and its members continue to support specification of high resolution climate data requirements via their existing programmes. These activities typically include user consultation on space segment missions, user engagement on climate data production programmes and cooperation with users in provision of  operational climate services. High-resolution data, together with various types of non-satellite data, continue to play a pivotal role in climate services as exemplified by the WMO report supplement to the Strategy Towards an Architecture for Climate Monitoring from Space.

### Climate Indicators

Development of indicators of climate change (GCOS Action G3) and Indicators for Adaptation and Risk (GCOS Action G4)

Establishment of Climate Indicators

Space Agencies are aware of the need to support the building of indicators of climate change. Many of the underlying climate data records are already available. What needs to be enhanced is a more systematic and correct consideration of uncertainty estimates.

Space Agencies are aware of the need to support the building of indicators for tracking climate change, based on the production of climate data records, including the systematic catering of uncertainties. To this end, planetary vital signs should be reliable, simple, have a long term history, reflect a range of possible symptoms of climate change, integrate as far as possible many wider effects of change, be easily interpreted and conversely difficult to misinterpret, either by accident or intent.

In addition, the Paris Agreement has led to the need for a new comprehensive set of climate indicators that should be forward-looking, i.e., aid decision-makers in assessing future climate change impact and the effects of measures to adapt to and mitigate impacts. Such indicators will rely on satellite and other observations, as well as modelling and forecasting capabilities, and are yet to be agreed internationally.

A number of parameters for tracking climate change such as sea level, snow cover, temperature, precipitation, ocean heat content and the cryosphere are covered by the WMO Statement on the Status of the Climate and supported by GCOS, and a WMO-GCOS position on indicators of climate change is established. Many CEOS and CGMS agencies will continue to enable CDRs critical for the formation of climate change indicators. Some ECVs are depending on multi-agencies inputs, e.g., precipitation that needs a multitude of measurements provided by several CEOS and CGMS agencies. For those WGClimate will foster multi-agency activities to deliver high quality climate data records that stand up to the needs for detecting and understanding climate change.

Architecture – From Satellite Missions to Services

Satellites are critical in providing the observations of numerous such parameters. For example, arctic sea ice extent has been measured since the late 1970s by passive microwave imaging systems and for the last 20 years or so by active radar imaging sensors also. In addition, dedicated satellites (Icesat, Cryosat) with specific relevant technologies have allowed measurements not only of the extent, but also the thickness and hence total volume of sea ice to be made. Sea level is now measured most accurately, and globally, by a series of precise altimeters; these measurements go back over 20 years and will continue into the future due to the Sentinel 6 series of missions.

Programmatically, the ESA extension to the Climate Change Initiative (CCI+, 2017 to 2026) includes continued research and development of ECV datasets tightly associated to holistic parameters, including sea level and sea ice. The programme extension also includes work on new ECV development including snow cover. The programme is similarly commited to cross-ECV research and development, and therefore aligning itself to the holistic perspective of climate change demanded by the Paris agreement and GCOS.

Climate data record development is also geared by specification of indicators by GFCS and climate service providers. WGClimate and its agencies plan to continue dialogue with international and national climate service providers, with the aim of the space agencies highlighting, optimizing and shaping their climate data production to address the emerging needs of climate indicators. In this respect the European Commission, ECMWF, ESA and EUMETSAT, for instance, are programmatically aligned to symbiotically address each other’s needs, and this includes consideration of ECV product uncertainty in production of climate change indicators. WMO is planning to enhance its interaction with users of satellite-based CDRs and ECV products, as a contribution to the Architecture for Climate Monitoring from Space, the GFCS Climate Services Information System and User Interface Platform. These are being established to facilitate the dialogue between the providers of observations and monitoring products, climate modellers, and service providers.

In context to the development of climate services to express the changing climate, provision of climate services for responding to adaptation needs and growing risk are equally crucial. Already embedded and proactive within WGClimate, users of satellite-based climate data records and climate service providers will be performing a more pronounced role in the future in the working group in light of the Paris Agreement. This will benefit the highlighting, aligning and shaping of climate data products in support of downstream climate services.

The vehicle for this amplified collaboration between space agency climate data providers, direct users of satellite climate data records, and climate service providers is the Architecture for Climate Monitoring from Space. The architecture, which logically and physically links space derived climate infrastructure and data to climate services and policy decision making support, has enabled productive dialogue across the variety of players within WGClimate.

The expanded role in WGClimate of WMO, the European Commission, ECMWF and other parties involved in climate services for adaptation and risk will include leading consultative roles in the following :

- addressing the shortfalls in the ECV Inventory between status of ECV datasets and their associated ECV requirements in the GCOS Implementation Plan, therefore providing valuable climate adaptation and risk insight on the consequences of those shortfalls.

- mapping ECV Inventory records to any new indicators of climate adaptation and risk, and consequently informing space agencies in the prioritisation of space resources to optimally address downstream needs in light of such indicators.

- raising governmental, intergovernmental and non-governmental policy needs which emerge by means of new indicators of adaptation and risk, therefore supporting the development direction of the architecture.

These activities will be systematically addressed through the WGClimate development of the architecture, including the continued development of the ECV Inventory. Additionally, the ECV Inventory, provides future opportunities for intelligently and conveniently interrogating the mapping between climate data records  and adaptation services.

## The Broader Relevance of Climate Observations

This section addresses GCOS-200 Part I §4 (“The Broader Relevance of Climate Observations”). Although Action G5 is neither directly nor indirectly actionable by space agencies, WGClimate will remain vigilant in identifying opportunities for maximising benefit from ECVs in implementing the GEO SDG process. Any such insight will be shared with GCOS and SBSTA through existing reporting cycles.

With the adoption in September 2015 by the United Nations of the 2030 Agenda for Sustainable Development, a set of 17 “Sustainable Development Goals” (SDGs) were been defined to be achieved in each country by 2030. Space Agencies, such as ESA, have confirmed their capabilities in the explicit support of these.

WGClimate recognizes the significant value in supporting the SDGs, both to inform, and be informed by, GCOS and SBSTA. This is particularly urgent in context to the Paris Agreement.

## Consistent Observations Across the Earth System Cycles

This section addresses GCOS-200 Part I §5 (“Consistent Observations Across the Earth System Cycles”).

The emergent need for holistic indicators of climate change following the Paris Agreement has magnified the need for WGClimate to adopt a similarly holistic forming to its climate data products and services, so informing observations across the Earth System cycles.

The analysis of the ECV Inventory content shows that many ECV Products needed to quantify the energy, water and carbon cycles are already provided from satellite data. CEOS and CGMS agencies already provided a couple of energy flux products at the top of atmosphere and surface (radiation and heat fluxes) before those became ECV Products.

WGClimate member agencies make also attempts to demonstrate the usefulness of satellite products for considering energy and water budgets, e.g., the NASA Measures programme. In addition, a current ESA CCI activity includes the objective of closure to sea level budget. The Sea Level Closure Budget project intends to analyse in an integrative context recent results obtained by the ESA CCI programme for the sea level, glaciers and ice sheets CCI ECVs, with additional account for assessments of the ocean thermal expansion component which utilize the CCI Sea Surface Temperature ECV. Studying the sea level budget and the ocean mass budget with accurate climate records will bring major scientific benefits – adding confidence to satellite-based assessments of climate change, shedding light on missing or poorly known processes such as land hydrology that remain a challenge for Earth Observation, or heat uptake by the deep ocean.

Quantifying the global carbon cycle is essential to understanding many of the dramatic changes taking place in the Earth system, particularly those resulting from the burning of fossil fuel and land-use change. Forests absorb, store and release large amounts of carbon, therefore, they are a key component of the carbon cycle. Despite this crucial role, forest biomass is poorly quantified in most parts of the world and is an ECV currently labelled missing in the ECV Inventory. Responding to this challenge, the ESA Biomass mission sets out to provide measurements of forest biomass and forest height. This urgently-needed data will shed new light on the state of our forests and how they are changing, and, in turn, advance our knowledge of the global carbon cycle. Reliable knowledge of forest biomass also underpins the implementation of the UN Reducing Emissions from Deforestation and forest Degradation, REDD+, initiative – an international effort to reduce carbon emissions from deforestation and land degradation in developing countries.

WGClimate will continue to actively support GEO carbon actions and strategy.

## Capacity Development and Regional and National Support

This section addresses GCOS-200 Part I §6 (“Capacity Development and Regional and National Support”).

The activities of WGClimate and its members are not entirely congruent with those GCOS actions focused on capacity development as defined by the GCOS Implementation Plan. Nevertheless, WGClimate can, and will, unreservedly support GCOS in its implementation of a GCOS communication strategy (Action G9), by raising awareness of the GCOS Implementation Plan and publicizing the need for sustainable climate observations, particularly for developing countries, in its own space agency communication strategies.

WGClimate member agencies are active are active in supporting capacity building initiatives focussed on climate services through the provision of access to climate data records and information, related user training and project management support. For instance, EUMETSAT has supported Africa through a Memorandum of Understanding signed with the African Union Commission and in the framework of the MESA programme funded by the 11th European Development Fund. EUMETSAT has in parallel supported the formulation of new projects expected to start in 2017-2018, i.e. the GMES and Africa and the ACP Climate Services projects, funded by different EU instruments.

Most agencies are strengthening their communication of climate monitoring activities, and these can be harmonized where possible with the GCOS communication strategy.

Building capacity of users to ingest satellite-based climate data and products into their operations and to interpret uncertainty is urgently required in many parts of the world where the competencies and infrastructure required are no match for the surging demand for high-quality climate information and services. The WMO-CGMS Virtual Laboratoy for Education and Training in Satellite Meteorology (Vlab) is establishing a programme of virtual climate roundtables where experts and users, for example from WMO Regional Climate Centres, can interact regularly and address region-specific capacity needs. The European Commission through its ACP programme is implementing programmes to foster the uptake of satellite data and derived information.

## Calibration and Validation of Satellite Data, Assessment and Intercomparison

Ground-based observing networks of sufficient quality and coverage (in terms of parameters, temporal and geographic space) are very important for validating satellite-based records, and for fully reaping the benefits of space-based systems. Gaps in such networks have been identified for most ECVs. Space agencies should step up their engagement with the operators of ground-based networks to address network gaps. Network design priorities should be established where appropriate, for example using Observing System Simulation Experiments or collocation studies.

Space agencies will develop a comprehensive assessment of the adequacy of surface-based networks for validation of climate data records, including a gap analysis and the resources required to address issues related to the sustainability and geographic coverage of surface-based validation networks. This should build on work undertaken e.g., within the CEOS WGCV and the European GAIA-CLIM research project.

Space agencies in collaboration with the operators of surface-based networks will invest in collocation studies demonstrating the value of using multiple data sources for estimating the uncertainty in atmospheric profiles.

Agencies continue to support the intercomparison of satellite data and derived products through international science working groups which unite leading experts in the field, and other users of satellite data. Intercomparisons foster product understanding including uncertainties, and guide the uptake of satellite data by users.

# Response to Part II : Detailed Implementation

## Overarching and Cross-Cutting Actions

### Planning, Review and Oversight

Development of an ECV Inventory (GCOS Action G11) and Gap Analysis (GCOS Action G12)

The objectives associated with ECV Inventory development are intrinsic to the fulfilment of the core objectives assigned to WGClimate in its Terms of Reference, and form a pivotal asset in the implementation of the Climate Monitoring Architecture. The development of the ECV Inventory was implemented applying a cyclic approach as indicated in Figure 2 that was tied to the term of the Chair of the WGClimate.

The ECV Inventory provides a repository of verified information for the characteristics of two types of ECV Climate Data Records:

- Climate data records that already exist. This forms the current component of the inventory;

- Climate data records that do not currently exist, but are firmly planned to be delivered as part of an already approved programme. This forms the future component of the inventory.

A first version of the ECV Inventory had been developed in 2015 followed by a major redevelopment during the last two years that resulted in the ECV Inventory Version 2.0 which forms the basis for this report. ECV Inventory Version 2.0 describes the status on 31 December 2016 versus the 2011 GCOS Satellite Supplement but contains additional data records matching new ECV Products in the updated GCOS IP. WGClimate is committed to continue gradual updates of the ECV Inventory including an analysis of the content versus the GCOS needs. This analysis leads to recommendations and actions for CEOS and CGMS that allow an optimised planning for future satellite missions and constellations to expand the existing and planned ECV CDRs and to deliver the data records by single agency and multi-partner projects.

Besides the analyses performed by WGClimate, the ECV Inventory is seen useful to provide information to the users of climate data records, climate services and other applications on what is available for their work. For data providers and data record developers the Inventory can help to concentrate investments onto what is missing and to foster collaborative approaches at international level such as the WMO SCOPE-CM. In addition the ECV Inventory has the potential to become a resource for capacity building activities that rely on the access to climate data records. Relevant activities such as the WMO-CGMS Virtual Laboratory for Training and Education in Satellite Meteorology and CEOS Working Group on Capacity Building and Data Democracy have been informed about the publication of the ECV Inventory.

Entries for the ECV Inventory Version 2.0 have been received from 10 CEOS and CGMS Agencies. The entries are based on a Questionnaire that provides traceability to the GCOS principles, guidelines and requirements. The preparation of the ECV Inventory content involved almost 100 individuals which shows the high engagement of the agencies to provide high quality information to the ECV Inventory.

Figure 2 : Cyclic approach for the establishment of the ECV Inventory, the analysis of its content leading to the identification of gaps and derivation of recommendations and actions to mitigate such gaps.

Table 1 provides the number of climate data records in the ECV Inventory with a very large complement for the Atmosphere and smaller numbers for land and ocean. A first analysis on gaps related to ECVs and individual ECV Product as defined by GCOS (GCOS-200) reveals that for two (above ground biomass and sea surface salinity) no entry exist in the current and future part of the Inventory. As this reflects the status on 31 December 2016 we know today that both missing ECVs will be addressed by ESA in the future. In addition, no entries exist for lightning and ocean surface currents because the Inventory was established against the 2011 Satellite Supplement.

| Domain | Total | Current | Future | Total (Cycle #1) |
| --- | --- | --- | --- | --- |
| All | 913 | 496 | 417 | ~200 |
| Atmosphere | 658 | 376 | 282 | N/A |
| Land | 135 | 56 | 79 | N/A |
| Ocean | 120 | 64 | 56 | N/A |

Table 1 : Number of data record entries in the ECV Inventory for Cycle#2

In terms of ECV Products with respect to the 2011 Satellite Supplement in the current data holdings 3 ECV Products are missing for atmosphere (tropospheric ozone profiles, NO2 tropospheric column, SO2 and HCHO tropospheric columns), 5 are missing for the terrestrial part (fire radiative power, ice sheet elevation data and mass change, areas of Lakes and above ground biomass) and two for ocean (sea surface salinity and wave height).

The number of missing ECV Products in the future part of the ECV Inventory is shown in Table 2

| Atmosphere | Land | Ocean |
| --- | --- | --- |
| Ozone Profile in Upper Stratosphere and Mesosphere | Elevation Data | Ocean Colour - Chlorophyll-a Concentration |
| Ozone Profile in Upper Troposphere and Lower Stratosphere | 2D Vector Outlines, Delineating Glacier Area | Ocean Surface Stress |
| Tropospheric Ozone Profile | Groundwater Volume Change | Sea-surface Salinity |
| Tropospheric CH4 Column | Ice Shelves |  |
| Stratospheric CH4 Profile | Soil Moisture - Freeze/Thaw |  |
| Aerosol Single-scattering Albedo | Fire Radiative Power |  |
| Aerosol-layer Height | Moderate-resolution maps of Land-cover Type |  |
| Aerosol-extinction Coefficient Profile | Above-ground Biomass |  |
| NO2 Tropospheric Column |  |  |
| SO2, HCHO Tropospheric Columns |  |  |
| CO Tropospheric Column |  |  |
| Solar Spectral Irradiance |  |  |
| Total Solar Irradiance |  |  |

Table 2 : Missing ECV Products in the future part of the ECV Inventory.

Due to the large number of data records the gap analysis for the ECV Inventory Version 2.0 has not been finalised and a first comprehensive gap analysis report will become available in spring 2018. This will also contain specific recommendations and actions to mitigate found gaps.

### Data Management, Stewardship and Access

Open Data Policies (GCOS Action G15)

Openness of space-derived climate data is crucial for the maximal uptake of such data by the user communities, for supporting unhindered collaboration in climate science, and promoting confidence in the data through transparency. Moreover, the integrity and quality of climate data products from space can only be fully realized when data from instrument to final climate data record along the traceability chain is open.

WGClimate has systematically collected and consolidated the climate data product access instructions of their constituent members by way of their ECV Inventory development. The ECV Inventory content show that an open data policy is todays standard for climate data records from satellites. Many space agencies have enhanced and progressed their open climate data policies and have planned for ambitious uptake of their open climate data. ESA, EUMETSAT, JAXA, NASA and NOAA run data portals that ensure the provision of open, free and easily accessible ECV data records. Other space agencies are on their way to develop their data distribution in similar directions.

This activity is complemented by GEO’s mission to build the Global Earth Observation System of Systems (GEOSS) supported through a Common Infrastructure (GCI) and the GEOSS Portal, a single Internet access point for users seeking data, imagery and analytical software packages relevant to all parts of the globe. The GEOSS Portal brokers numerous climate data products.

Meaningful policy on open climate data is predicated on open and freely available tooling to act on the data. An Open Source cross-ECV toolbox is being released under ESA procurement in late 2017, with plans for its continued service confirmed (CCI+, 2017 to 2026). Space agency investment in open climate data tooling has generally grown strongly the last 5 years, for example at EUMETSAT, NASA, and NOAA, and evidence exists of its continued role in enabling open climate data.

Use of Digital Object Identifier for data records (GCOS Action G20)

The ECV Inventory and the continued development of the Architecture for Climate Monitoring, will be capitalized on to assess which climate data records have missing DOIs. This practice will commence in Inventory Development Cycle #4 (2020 to 2022), with owners of non-DOI datasets being notified of this incomplete status, and accompanied by request for DOIs. This activity will similarly take place to ensure new DOIs for updated versions of existing data records in the inventory.

The emerging need for DOIs on climate data sets has emerged over the last 5 years, and they have become the de facto method for uniquely identifying data records and other research assets. The content of the ECV Inventory demonstrates that for more than 80% of the climate data records the DOI is already in use. This is also recognized by major programmes of WGClimate member agencies to deliver climate data records such as ESA, EUMETSAT, NASA, and NOAA.

### Production of Integrated ECV Products

Preservation of early satellite data (GCOS Action G26)

Identification, recovery and curation of early satellite data is a necessary activity for ensuring full capitalization of space-derived earth observation data. These data are important because the coverage with satellite data already started in the mid-1960s. Major thrust in this sector is the usage of historical satellite data for global reanalysis. In Europe the series of dedicated research projects has fostered very much data rescue and reprocessing activities for historical satellite data. Space agencies such as NASA run dedicated programmes to make historical data available online for analysis and usage. In Europe the Copernicus Climate Change Service will have a dedicated project on satellite data rescue that will be supported by EUMETSAT.

The future usage of such data for the delivery of ECV CDRs will be reflected in upcoming versions of the ECV Inventory. This will include engagement of the CEOS Working Group on Information Systems and Services (WGISS) to ascertain information on current and planned activities on the recovery of early space-derived data potentially relevant to climate data processing. WGClimate will use this information to perform an overlap analysis against the data needs of space agencies producing climate data products, on gaps existing between climate data products and GCOS requirements.

The overlap analysis may reveal opportunities for gap-filling in the ECV Inventory. WGClimate will engage relevant space agency climate teams to inform them of these opportunities and broker a relationship between WGISS and space agency team to support the gap-filling.

### Ancillary and Additional Observations

Improve Gravimetric Measurements form Space (GCOS Action G31)

[#]

Improved Bathymetry (GCOS Action G32)

[#]

## Atmospheric Climate Observing System

The Space Agency response to the GCOS IP 2016 Atmosphere-related actions is structured into three aggregate topics, covering all the thirteen Actions with direct relevance to Apace Agencies. In addition, six Actions mainly address individual surface-based networks which are “non-space” in nature but important to Space Agencies for the purposes of calibrating instruments and validating products. Action A31 calls for concerted action by Space agencies to engage existing networks of ground-based remote sensing sites for validating satellite-based data. A generic Space Agency action is proposed to address these concerns (see section 3.5). The following provides a short summary of the detailed responses by Space agencies to each relevant GCOS IP 2016 Atmosphere action.

### Atmospheric Dynamics, Energy and Water Cycle Components

Reprocessing of atmospheric motion vectors (GCOS Action A19)

Tracking atmospheric motion is important for predicting weather systems, and for understanding atmospheric dynamics. For this purpose, satellite-based upper-air winds are derived at various scales and heights, with demonstrated impact on NWP and for climate applications.

Reprocessing of upper-air wind vectors derived from historical geostationary imagery (atmospheric motion vectors) is important for NWP and climate applications. Several Space agencies have reprocessed parts of their archived data ranging back to the mid-1990s, coordinated internationally within a SCOPE-CM project. These activities will continue using state-of-the-art processing algorithms, encompassing AMV data from the early 1980s up to the present. Winds derived from polar-orbiting imagers and sounders are another important component of the upper-air wind record, especially in high latitudes.

Implementation of space-based wind profiling system (GCOS Action A21)

To pioneer 3D upper-air wind profiles based on lidar measurements, the ESA ADM-Aeolus mission has been developed, with launch planned in early 2018. ESA and EUMETSAT are planning a user workshop to facilitate user preparation, and to discuss a potential follow-on mission, pending success of this exploratory satellite.

Continuity of global satellite precipitation products (GCOS Action A25)

Precipitation is arguably the most important ECV regarding direct societal impact. Satellite-based methods to estimate and quantify precipitation globally are improving. Concerns about the continuity of key missions for deriving global precipitation products (conical-scanning and other passive microwave (MW) imagers, precipitation radars) are recognized by several Space agencies, as well as within the CEOS Precipitation Virtual Constellation and the International Precipitation Working Group (IPWG). The value of dedicated MW sounding to improve coverage of rainfall events in the tropical regions has been demonstrated and will be extended. Novel approaches based on precipitation radars on cubesats and microwave sounders in geostationary orbit are being studied. In addition, the GPM precipitation radar mission provides a critical calibration standard and generates unique databases of precipitation types; GPM continuity is being explored by NASA and JAXA. IPWG continues to pursue intercomparison and validation of global precipitation products using a common protocol.

Continuation of precipitation climatologies such as GPCP is of primary importance to detect trends and anomalies in precipitation; coordination and mobilization of resources should occur under SCOPE-CM.

Development of methodology for consolidated precipitation estimates (GCOS Action A26)

IPWG also promotes the development of methodologies to improve precipitation estimates based on various data sources (IR and MW satellite data, surface-based, models); to facilitate user uptake, IPWG could do more to consolidate the various approaches. Reprocessing of global satellite precipitation products is underway within several agencies, but requires enhanced international coordination and cross-sensor consistency. To this end, cross-calibration of MW sensors is being addressed by the GSICS and GPM/X-CAL groups (see also A16).

Space agencies will act to facilitate user uptake of precipitation products in societal applications. IPWG continues to foster the development of methodologies to consolidate precipitation estimates based on blended products using satellite, surface-based (gauge, radar) and model data.

Dedicated satellite Earth Radiation Budget mission (GCOS Action A27)

The Earth’s energy balance (A27) and thus its climate is determined by the amount and distribution of the incoming solar radiation absorbed by the Earth and the outgoing longwave radiation (OLR) emitted by the Earth. The top-of-atmosphere (TOA) Earth radiation budget can only be measured from space, and continuity of observations is critical to interpret changes in surface and upper-air warming, and to constrain climate change predictions and projections. Total solar irradiance and spectral solar irradiance have been monitored since 2003 and are expected to continue, partly using the ISS as host. TOA reflected shortwave and outgoing longwave radiation have been monitored since the 1980s; continuity of measurement is secured through a payload on JPSS-1, but uncertain beyond 2024. Space agencies need to address the potential gap in measuring TOA upwelling radiance to enable continuity in deriving the Earth radiation budget.

Space agencies will act to address the potential gap in measuring TOA upwelling radiances (SW, OLR) to enable continuity in deriving TOA Earth radiation budget beyond 2024.

Lightning (GCOS Action A29)

Lightning, a recent addition to the list of ECVs, can be used as a proxy for monitoring severe convection and hence precipitation, and for improving estimates of severe storm intensity. Ultimately the data could be assimilated in NWP models and climate reanalyses to improve the representation of severe storms. The first lightning mappers in geostationary orbit operated by NOAA and CMA are entering operations in late 2017, and more will be added with the launches of GOES-S and MTG-I. Space agencies will distribute products from these satellites (lightning events, groups, and flashes) in near real-time, and are expected to perform reprocessing of the new GEO-ring-based data in due time to extend the 20-year low-Earth orbit lightning dataset that is based on heritage lightning imagery from the TRMM mission. In developing application modules and proxy datasets, space agencies can assist as needed in the refinement of user requirements, as the application of lightning data matures.

### Atmospheric Composition Including Greenhouse Gases

A number of atmospheric constituents have an important role in climate forcing and feedbacks, such as water vapour, CH4, CO2, O3 and aerosols. There is paramount importance for countries to monitor sources and sinks of greenhouse gases in the context of the 2015 UNFCCC Paris Agreement. Other gases, such as NO2, SO2, HCHO, NH3 and CO, are required to detect and attribute changes in ozone and aerosol in the troposphere and lower stratosphere, and for air quality monitoring.

Water vapour and ozone measurement in upper troposphere and lower and upper stratosphere (GCOS Action A30)

Global high vertical resolution measurements of water vapour and ozone in the UT/LS by limb observations are essential for monitoring UT/LS fluxes of these constituents. This is important to better quantify their cycle and to better understand the causes of upper-air temperature trends, and related climate forcing/feedback mechanisms. There is currently no plan to fill the potential gap in ozone limb profiling between the Suomi-NPP and JPSS-2 missions. No follow-on to MLS on Terra/Aura or SMR on Odin is planned to provide MW limb profiling of water vapour, ozone and other constituents. No follow-on to OSIRIS on Odin is planned to provide UT/LS scattering limb profiling of ozone and aerosols. No sustainment of solar occultation soundings of atmospheric constituents is planned (such as from ACE-FTS and ACE-MAESTRO on SCISAT, or SAGE-III on ISS). Fig. 9 in the GCOS IP shows that most missions operate well beyond their anticipated lifetime.

Space agencies will act to sustain limb-scanning satellite missions for measurements of profiles of water vapour, ozone and other species from UT/LS up to 50km altitude.

At the same time, GNSS-RO radio occultation-based measurements of atmospheric profiles should be maintained and their quality and coverage enhanced, given the demonstrated value of these data for NWP and climate applications.

Develop a repository of water vapour CDRs (GCOS Action A22)

Building on significant efforts by the GEWEX climate research community to build a globally recognized repository of consistent surface-based GNSS zenith total delay and total column water datasets, satellite agencies should contribute satellite-derived water vapour climate data records to this repository. In addition to committing to exchange GNSS-RO data internationally, space agencies will act to maintain quality and coverage of the GNSS-RO constellation.

Space-based measurements of CO2 and CH4 implementation (GCOS Action A35)

The complementary role of surface-based networks and satellite data for monitoring CO2 and CH4 within a comprehensive global observing system is increasingly accepted (cf. Actions A31 and A33). The quality of existing CO2 and CH4 satellite-based estimates has been assessed in the scientific literature as well as in operational contexts such as the European Copernicus Atmosphere Monitoring Service. Such estimates play an important role in quantifying the carbon cycle and to understand sources and sinks of these major GHGs.

Several Space Agencies plan a range of dedicated CO2 and CH4 missions (see Table below), with strong emphasis on better constraining anthropogenic carbon emissions in the atmosphere, and to improve emission inventories. To validate satellite-derived products, surface-based networks for CO2 and CH4 (A33) and other GHGs, such as N2O, SF6 and halocarbons, need to be maintained and their coverage extended (see Actions A31 and A36).

Space agencies, through CEOS and CGMS, will coordinate the space-based component of a global greenhouse gas observing system. Cross-linkage to the development of a global carbon monitoring system should be ensured (see Action T71).

| Instrument | Agencies | Launch date | XCO2, XCH4 | Instruments | Precision | Sampling (km) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Missions already launched |  |  |  |  |  |  |  |
| SCIAMACHY | DLR / NSO / BelSpo / ESA |  | CO2 Total Column CH4 Total Column | UV/VIS/NIR/ SWIR –spectrometer | 2.5 ppm 80ppb | 30x60 | Reuter et al. (2011), GHG-CCI (2017) |
| AIRS | NASA | May 2002 | Mid-trop CO2 Mid-trop CH4 | TIR spectrometer | 2ppm | Diam. 15 | Maddy et al. (2008) Chaine et al. (2008) |
| IASI | CNES-EUMETSAT | Oct. 2006 | Mid-trop CO2 Mid-trop CH4 | TIR Spectrometer | 2ppm | Diam. 12 | Crevoisier et al. (2009a) |
| TES | NASA | 2004 | Mid-trop | TIR spectrometer | 10ppm | 5.3 x 8.3 | Kulawik et al. (2010) |
| GOSAT | JAXA / MOE / NIES | Feb; 2009 | CO2Total Column CH4 Total Column Mid-trop CO2 Mid-trop CH4 | SWIR spectrometer and TIR | 2 ppm 13 ppb | Diam. 10,5 | Butz et al, (2011) Yoshida et al. (2013), GHG-CCI (2017) |
| OCO-2 | NASA | Jul; 2014-… | CO2 Total Column | SWIR spectrometer | 1.5 ppm | 1,29 x 2,25 | Crisp et al. (2017) ; Wunch et al. (2017) |
| Tansat | MOST / CAS / CMA | Dec. 2016- ... | CO2 Total Column | SWIR spectrometer | 4 ppm | 2x2 |  |
| Decided missions (in phase B, C or D) |  |  |  |  |  |  |  |
| MERLIN | CNES / DLR | 2021 | CH4 Total Column | SWIR Lidar | 20 ppb | 50 km | https://merlin.cnes.fr |
| OCO-3/ISS | NASA | 2018 | CO2 Total Column | SWIR spectrometer | 1ppm | 2 x 2 | Basilio et al. (2013) |
| GOSAT-2 | JAXA / MOE / NIES | 2018 | JAXA/MOE/NIES | SWIR and TIR spectrometer | 0.5 ppm (CO2) 5 ppb (CH4) | Diam. 9.7 |  |
| Sentinel 5P | ESA / NSO | 2017 | CH4 Total Column | SWIR spectrometer | 5 ppb | 3.5*7 | https://sentinels.copernicus.eu/web/sentinel/missions/sentinel-5p |
| Sentinel 5 | ESA / EUMETSAT | 2021 | CH4 Total Column CO2 Total Column as scientific product only | SWIR spectrometer | 4 ppb | 7.5*7.5 | https://sentinels.copernicus.eu/web/sentinel/missions/sentinel-5 |
| MICROCARB | CNES | 2020 | CO2 Total Column | SWIR spectrometer | 1ppm | 4,5*9 (2*2 demo) | https://microcarb.cnes.fr |
| GF5/GMI | CNSA | 2017 | CO2 Total Column CH4 Total Column | SWIR spectrometer |  | Diam. 10.3 |  |
| GAS/Feng Yun 3D | CMA | 2017 | CO2 Total Column CH4 Total Column | SWIR spectrometer | 4 ppm | 10x10 |  |
| HIRAS/Feng Yun 3D/E/F/G | CMA | 2017-25 | Mid-trop CO2 Mid-trop CH4 | TIR spectrometer |  | 16x16 |  |
| GASIII/Feng Yun 3G | CMA | 2021 | CO2 Total Column CH4 Total Column | Grating Spectrometer | 4 ppm | 3x3 |  |
| eoCARB | NASA | 2021 | CO2 Total Column CH4 Total Column | SWIR spectrometer | 1ppm CO2 4 ppbv CH4 | 4.5 x 5.5 | https://eospso.gsfc.nasa.gov/missions/geostationary-carbon-cycle-observatory-evm-2 Polonsky et al. (2014) |
| GHG Sat | Canada | - | Total Column | - | - | - | http://www.ghgsat.com/ |
| Sentinel 7 – GHG Constellation (3 LEO satellites) | ESA / EC | 2024-26 | CO2 Total Column CH4 Total Column | SWIR spectrometer | 0.5 ppm CO2 4 ppb CH4 | 2 x 2 (still tbc) |  |

The references are as follows :

- Chahine, M. T., L. Chen, P. Dimotakis, X. Jiang, Q. Li, E. T. Olsen, T. Pagano, J. Randerson, and Y. L. Yung (2008), Satellite remote sounding of mid-tropospheric CO2,  Geophys. Res. Lett., 35, L17807, doi:10.1029/2008GL035022.

- Crisp, D., Pollock, H. R., Rosenberg, R., Chapsky, L., Lee, R. A. M., Oyafuso, F. A., Frankenberg, C., O’Dell, C. W., Bruegge, C. J., Doran, G. B., Eldering, A., Fisher, B. M., Fu, D., Gunson, M. R., Mandrake, L., Osterman, G. B., Schwandner, F. M., Sun, K., Taylor, T. E., Wennberg, P. O., and Wunch, D.: The on-orbit performance of the Orbiting Carbon Observatory-2 (OCO-2) instrument and its radiometrically calibrated products, Atmos. Meas. Tech., 10, 59-81, doi:10.5194/amt-10-59-2017, 2017.

- Basilio, R., et al. “ Systems Design And Implementation Of The Proposed Orbiting Carbon Observatory-3 On The International Space Station’s Japanese Experimental Module.” Science and Technology Series 114.20130000: 321-323, 2013.

Monitoring of aerosol properties (GCOS Action A39)

Aerosols remain a major source of uncertainy in estimating radiative forcing which is important for driving climate models and climate change projections. The latest IPCC AR5 report estimated that the direct aerosol radiative forcing (DARF) is -0.35±0.5 Wm-2. However, there remains considerable observational as well as modelling evidence that the actual uncertainty is a factor of two to four larger. The large uncertainty of DARF is caused by the combination of uncertainty in aerosol optical depth (AOD) retrieval from satellite observations, and the lack of accurate global characterizations of three-dimensional aerosol optical and microphysical properties. Current satellite instruments provide a global measurement of AOD and qualitative aerosol-type classification, but lack the capability of providing sufficient microphysical details to reduce the uncertainty in DARF and discriminating the anthropogenic components from the natural components. Space agencies should aim to support the overarching goal to reduce the uncertainty in DARF by mapping the global 3D AOD distribution with an accuracy of 0.02 at the mid-visible wavelength. The retrieval of aerosol properties (types, shapes, size distributions) is also of high importance. A number of current and planned missions, feeding advanced multi-sensor analysis techniques, will contribute to progressing towards those objectives but more needs to be done.

Clouds and aerosols remain one of the largest factor of uncertainty in quantifying climate forcing and feedback. Satellite missions such as Calipso and CloudSat have revolutionized the understanding of 3D vertical profiles of clouds and aerosol layers, their composition (liquid/ice, chemical) and evolution over time. Continuity of space-based lidar observations beyond the Calipso and EarthCARE-type, and of limb scatter profiling observations (see A30) is essential for supplementing passive nadir-viewing sensors in terms of vertical resolution and coverage of the lowest atmospheric layers, however, mission continuity for aerosol and cloud vertical profiling beyond Calipso and EarthCARE is uncertain.

Space agencies will act to coordinate the planning for a successor cloud and aerosol lidar mission beyond Calipso and EarthCARE.

Ground-based measurements (i.e., Aeronet, GALION) of AOD and aerosol optical property retrievals are instrumental to validate the satellite retrievals of AOD and provide climatology for aerosol types (see Action in response to A31). Sub-orbital measurements will also play a role in better characterizing the microphysical properties of aerosols.

Fundamental Climate Data Records and Climate Data Records for greenhouse gases and aerosols ECVs (GCOS Action A32)

Many Space agencies in Europe, the U.S. and Asia run major programmes devoted to the generation of climate data records for GHG amount and aerosol (A32) loading based on heritage instrument records. Significant efforts are going into cross-calibration and reprocessing of sensor records. Several current missions are providing CO2 data, and there are plans to extend the CO2 and CH4 record by many agencies (see Table under Actions A35). In addition, solar-induced chlorophyll fluorescence (SIF) was first measured from space using high resolution spectral observations.  Now a global and long term SIF dataset is available for better understanding of CO2 uptake. Continuation into the future to construct a long-term record for trend detection will require inter-calibrating and connecting heritage satellite sensor measurements with new generation of sensors, such as hyperspectral sounders, multi-spectral imagers, and multi-spectral multi-polarization instruments.

Ozone network coverage (GCOS Action A37)

Total column and profiles of ozone (A37) are routinely monitored using surface-based UV spectrometers coordinated globally through WMO Global Atmosphere Watch (GAW). Satellite-based retrievals of ozone parameters are a key contributor to the ozone climate record, and nadir-viewing ozone missions are secured in the long-term. Space agencies should help addressing the decline in GCOS global baseline ozone networks through supporting network design experiments (see also A31).

Space agencies will support network design experiments (OSSE-type) to optimize the locations of Dobson-Brewer stations that are necessary for characterizing stratospheric ozone for climate research and applications (see section 3.5 and Action A31).

Continuity of products of precursors of ozone and secondary aerosols (GCOS Action A40)

For monitoring precursors of ozone and aerosols, a series of LEO and GEO orbiting satellite missions will ensure long-term satellite measurements of the precursors of ozone and secondary aerosols from space. Ground-based measurement networks (e.g., GAW) will insure the same for in-situ measurements, and Space agencies are supporting some of these activities. Observations of precursors of aerosols and ozone, such as NO2, SO2, HCHO, NH3 and CO, are required to detect and attribute changes in ozone and aerosol in the troposphere and lower stratosphere (see A30). They are also important variables for air quality monitoring.

The generation of emission inventories is currently mainly based on in-situ data (e.g. EMEP, REAS), and Space agencies are supporting the development of services to demonstrate the utility of satellite measurements for establishing more consistent emission databases.

### Satellite Data Calibration, Validation and Reprocessing

This sub-section addresses Actions A11, A15, A16, A18, A31, and A36 related to enhanced satellite calibration and inter-calibration, data validation, and reprocessing of hyperspectral satellite records.

Implementation of satellite calibration missions (GCOS Action A16)

For “anchoring” the satellite-based climate record against an absolute standard, a dedicated in-orbit SI-traceable calibration mission is required against which other satellite instruments can be calibrated. This need was identified in the Architecture for Climate Monitoring from Space (Dowell et al., 2013). However, there are no firm plans by space agencies for such mission, which would underpin and operate in the context of the WMO Global Space-based Inter-Calibration System (GSICS). Putting inter-calibration of VIS and IR sensors within GSICS on a more solid footing, and expanding GSICS to include passive MW sensors are interim objectives in the absence of progress with a fully dedicated satellite calibration mission. A partnership between Space agencies and NMIs is required to foster the development of SI-traceable standards in the MW spectrum at the level of uncertainty required for climate.

A partnership between Space agencies and relevant national metrology institutes (NMI) should be established to enable and promote the development of SI-traceable MW reference standards of sufficient quality to meet climate requirements.

Validation of satellite remote-sensing (Action A31), including: Operation of the BSRN (Action A11) Implementation of GRUAN (Action A15), N2O, halocarbon and SF6 networks (Action A36) and Aeronet (Action A39) .

Validating satellite-based climate data products is necessary to characterize their quality and uncertainty, which are key elements for fostering user acceptance and serving as feedback to satellite operators. Several Space agencies are investing substantially in surface-based networks for calibration and validation, such as BSRN (A11), NDACC, TCCON, GRUAN (A15), N2O, halocarbon and SF6 networks (A36), and Aeronet (A39), through science programmes or dedicated mission-based funding. However, despite these investments, many surface-based networks have insufficient resources to ensure sustained operations and to fill major geographical gaps (mainly in Africa, South America, Pacific region), and thus cannot adequately serve the needs for validation. Spatial gaps includes the oceans where surface-based measurements are sparse and often not adequately supported, for example, surface precipitation validation networks on research vessels or moored buoys. Collocation studies should be supported to demonstrate the benefits for estimating uncertainty using several data sources (satellites, surface networks, model). Building on existing activities (e.g., CEOS WGCV), Space agencies will develop a plan how to strengthen the various surface-based networks to become a backbone for satellite data calibration and validation, and how to finance such measurements in all parts of the world. This requires on the one hand that some surface-based measurements become so-called fiducial reference measurements but also requires a long-term view on support and governance.

Space agencies will act, together with meteorological agencies and science institutions, to develop a plan for a comprehensive validation of climate data records using surface-based data, including the resources required to address sustainability and sufficient geographic coverage of surface-based networks for calibration and validation.

Space agencies will act in collaboration with the operators of surface-based networks to invest in collocation studies demonstrating the value of using multiple data sources for estimating the uncertainty in atmospheric profiles.

Hyperspectral radiances reprocessing (GCOS Action A18)

Many Space agencies such as EUMETSAT, NASA and NOAA have commitments to reprocess existing sensor data, including from their hyperspectral sounder instruments. Several activities are underway and efforts focus on more efficient, consistent and harmonized reprocessing across agencies, with international coordination envisaged through SCOPE-CM. This will result in a 20+ year FCDR of hyperspectral sounder radiances, representing a rich data resource for climate applications such as reanalyses. High-resolution IR sounders are expected to continue over the coming decades, improving the vertical resolution of satellite-derived upper-air water vapour and temperature profiles, and thus enhance monitoring of the atmosphere.

## Oceanic Climate Observing System

This section addresses GCOS-200 Part II §4 (“Oceanic Climate Observing System”).

The CEOS agency response to requirements for Ocean variables is structured into five aggregate topics, covering all the GCOS IP actions identified with a CEOS-agency relevance. The generic action O6 to continue to develop technology for satellite systems among others is implicit to all five topics. Some actions (O9, O11, O17, O23, and O29) are essentially non-space in content, but are addressed in the GCOS IP to CEOS (inter alia) because of their importance to integrated space-in-situ observation and to satellite calibration; these non-space actions are included in the response below.

### Physical – Sea Surface Thermodynamic State

Sea surface thermodynamic state includes activities addressing essential climate variables (ECVs) that quantify surface thermodynamic state (surface temperature and phase) and driving heat fluxes. This topic collects the CEOS agency response to actions O8 (sea surface temperature (SST) products), O17 (system for ocean surface heat flux observation), O29 (satellite validation, here for SST primarily), O31 (securing microwave SST missions) and O35 (sustaining sea ice products through active and passive microwave missions).

Satellite SST Product Development (Action O8), Ocean-surface heat-flux observing system (Action O17), Satellite SST (Action O31), Satellite sea ice (Action O35), In situ data for satellite calibration and validation (Action O29).

Mission Continuity and New Missions

Regarding sea surface temperature (SST), infra-red surface-imaging missions with ~1 km spatial resolution on polar orbiting platforms are internationally well secured for continuity over the next decade, with relevant meteorological and environmental monitoring programmes across several agencies.

SST observation from geostationary sensors is at somewhat coarser resolution (2 to 10 km) and supports resolution of the diurnal cycle of SST, which is increasingly understood to be of climatological relevance particularly in the tropics. Himawari 8 is operational and Himawari 9 is on standby to ensure continuity of observation over the W Pacific sector. The first generation of Geostationary Operational Environmental Satellites (GOES) with Advanced Baseline Imagers (ABIs) will become operational between end 2017 (GOES-16) and circa 2020 and will restore diurnally resolved day-time (split-window) SST capability for the west Atlantic and east Pacific sectors. Several international geostationary programmes with diurnal SST capability address other longitudes/sectors completing the necessary constellation securely for at least 5 years according to published plans, with Meteosat-8 presently covering ~40E until 2019.

A major concern for both SST and sea ice (SI) continuity of observation is the absence of plans across many agencies for low-frequency passive microwave radiometry (of the DMSP/SSMI (for SI) and JAXA/AMSR (for SST and SI) prior to 2022, while AMSR2 is nearing nominal end of life. There appears to be continuity of scatterometer missions, mainly directed at ocean vector wind determination but with SI capability, although with concerns about the consistency/stability of the climate data record over time from use of different techniques.

Product Evolution

Generation of long records of daily SST analyses is maturing, the main ongoing evolutions in progress being: improvement of true feature resolution at ocean mesoscales by methods that are more robust to the varying spatial resolution and gappy coverage of input data streams over decades; and development of multi-decadal climate data records of high observational stability and quantified uncertainty, with scientific challenges in maximizing the benefit for climate data from the early record (1970s and 1980s). Evolution of diurnally resolved SST analysis is at an earlier stage and requires development of more subtle understanding of the sensitivity of SST observation (diurnal amplitude biases) and of inter-satellite SST errors (since the cycle is constructed from multiple overpasses), as well as developing computational capacity to integrate GEO and LEO data streams for long-term reprocessed climate data. Coastal zone SST analyses with higher resolution (<300 m) are desirable for climate impacts in coastal and estuarine environments. CMA are developing 250 m regional SST (from FY3D) and the NOAA VIIRS and JAXA SGLI sensors have nadir 300 m and 500 m SST capacity respectively.

Sea ice products such as concentration and extent are well established. As long as they are supportable by the observing system, sea ice type, drift and emissivity products will continue to be improved and evolved. Operational ice surface temperature products have emerged and will continue to evolve, and should be developed as climate data records within CDR programmes.

Global ocean surface heat flux products must rely on methods that indirectly constrain the fluxes: available satellite observations have negligible direct sensitivity to the fluxes themselves and the near-surface (e.g. 10 m) air temperature & humidity used in bulk formulae. Continued progress underpinned by investment should be addressed to further characterizing the ability of combined satellite observations to constrain fluxes, and should be pursued by agencies including ocean surface heat flux within CDR programmes (Bentamy et al., 2017).

Product Generation and Inter-comparison

Agencies will continue to generate and distribute SST and SI products at L2 and L3 from polar and geostationary instruments to meet operational (NWP) needs and monitoring of climate variability. NOAA will continue to generate and develop polar-geostationary-blended SST resolving diurnal variability, which can be compared with a UKMO diurnal analysis product. EUMETSAT will support the CEOS SST Virtual Constellation (Group for High Resolution SST, GHRSST), within which inter-comparison for operational products, analyses and climate data records is co-ordinated on a best-efforts basis.

Reprocessing Needs

The EUMETSAT Ocean and Sea Ice Satellite Application Facility will provide reprocessing of the MSG/SEVIRI archive from MSG/SEVIRI (2004 to 2012), and global SI concentration from Nimbus-7/SMMR, DMSP/SSM/I and DMSP/SSMIS (1978 to 2015) and from EOS AQUA/AMSR-E and GCOM-W/AMSR2 (2001-2015), the latter activity in co-ordination with ESA’s climate change initiative (CCI) project on SI. ESA CCI will also provide a reprocessing and CDR of SST covering 1981 to 2016, with updates thereafter. CMA will reprocess FY3C VIRR SST, and JAXA will reprocess AMSR-E.

Validation, Metadata

Publicly available validation systems and data will continue to be provided by agencies, including NOAA’s iQUAM and ESA-funded Felyx. Towards traceability to Systeme International standards and improved absolute stability of observation, ESA will continue to develop the concepts and deployment of Fiducial Reference Measurements for surface temperature in international partnership.

### Physical – Sea Level and Surface Dynamic State

Sea level includes activities addressing the climate-scale changes in sea surface height (SSH) some dynamic variability having been addressed within sea surface dynamic state. This topic collects the CEOS agency response to actions O9 (upper level observing system), O13 (sea level observation systems) and O31 (altimetry systems for SSH). The GCOS IP specifically calls Space Agencies to maintain and develop a global sea surface height observing system from the observational and satellite networks for annual assessment of sea level and sea level rise. Further it requires to ensure continuous coverage from one higher-precision, medium-inclination altimeter and two medium-precision, higher-inclination altimeters. Regarding the satellite altimetry reference mission, there should not be any gap between successive missions and an overlap period of 6-9 months to inter-calibrate each other (example of TOPEX/Poseidon and Jason missions). Care is required not to introduce bias in the data record due to the change from pulse-width-limited altimeters and synthetic aperture altimeters. Sea surface dynamic state includes activities addressing ECVs that quantify ocean currents (horizontal) and sea state and the driving factor of wind stress. This topic collects the CEOS agency response to actions O12 (gridded current products), O16 and O34 (surface stress observations), O29 (satellite validation, for dynamic-state ECVs), and O33 (sea state from satellite). Theoretical and practical progress continues in estimating total and component surface current velocities by combinations of sea surface height, temporal evolution of features in SST and ocean colour, and/or in situ drifters and/or models.

Upper-ocean temperature observing system (Action O9), Sea level observations (Action O13), Satellite sea-surface height (Action O31). .Ocean current gridded products (Action O12), Ocean surface stress observations (Action O16), In situ data for satellite calibration and validation (Action O29), Satellite sea state (O33), Satellite ocean surface stress (O34).

Mission Continuity and New Missions

For what concerns Altimetry Reference High precision, Medium inclination Mission, European-US cooperation has ensured the continuity of the reference mission, without any gap, from Topex-Poseidon to Jason-3 (current). Sentinel-6A is planned to be launched in 2021 – in principle overlapping with Jason-3 for inter-calibration, the two successive S-6 with a planned life time of 5.5 years each fulfilling the mission continuity requirement without gap until 2030. Performance is expected to improve over the data record, e.g., an increase in precision is expected from SAR for Sentinel-6. Periods where two interleaved ground tracks are covered see an increase of the spatial resolution. A planned further contribution is China’s HY-2C (planned in 2019). Care is required to ensure that the long-term record is not biased due to the very different data processing and sampling of a SAR altimeter. The Sentinel-6A Posiedon-4 altimeter introduces a new altimeter mode of operation (interleaved) that allows both SAR and pulse-width-limited measurements to be made at the same time.

Past contributions to Altimetry Higher Inclination Missions include ESA ERS-1, ERS-2, ENVISAT, ISRO-CNES Saral-AltiKa. Current and planned European contributions are: Sentinel-3 satellites, comprising the A + B constellation followed by the C + D, gap-free from 2016 to 2030; and Cryosat-2, whose main goal is to determine fluctuations in the mass of the Earth’s major land and marine ice fields, and has demonstrated SAR observation above the ocean and contributed to high latitude coverage. The Sentinel A + B constellation will initially fly in a tandem phase enabling SAR and LRM altimeter inter-calibration following GCOS principles, before the B mission moves to an orbit plane configuration well-optimised relative to the A orbit for altimetry. Near identical precision to the reference mission is achieved using Synthetic Aperture Radar (SAR) mode observations. Sentinel-3 is the first satellite to operate a SAR altimeter 100% of each duty cycle. Chinese contributions in this category are HY-2A (2011-2016) and HY-2B (planned in 2018 for a 5 year life time).

The new swath altimetry mission SWOT (France-US-Canada-UK, planned in 2021) will characterize ocean mesoscale and sub-mesoscale sea surface height at spatial resolutions >15 km using interferometric SAR imaging techniques.  This mission will not fly in the reference orbit.

For what concerns Surface Dynamic State, several agencies have committed plans for continuity of missions for ocean dynamics satellites. Scatterometer observations have planned continuity over the coming 5 to 10 years, providing ocean vector winds and co-derived variables. Altimetry missions ensure continuity of significant wave height products.

Product Evolution

Regarding sea level, the requirement for a Level 2 product, i.e., the Geophysical Data Record (GDR), is for 1-Hz sampling of the along-track signal of sea level, significant wave height and wind speed. The scope of the products keeps reaching closer to the coastal zone through improved tracking schemes and increase of the along-track resolution, the latter enabled by SAR observation (CS-2, S-3, S-6). However, access to full calibration is required. L1a products are now emerging (standard products for Sentinel-3 SRAL) that allow reprocessing by multiple users and greatly facilitate algorithm development in the new SAR altimeter era, supporting innovation to maximize the quality of data. Wider work on L1a products should be continued for all future altimeters, to broaden the range of processing techniques that are tested and applied to the data, and encourage innovation in aspects such as uncertainty estimation.

Regarding sea surface dynamic state, further progress is required and intended on combined analyses of data streams each giving complementary perspectives on surface currents and sea surface state. A challenge for product evolution is the development of stable long-term climate data records from an evolving constellation of sensors, particularly going back in time to maximize the record and combining where available relevant wave parameters from SAR. One initiative in this domain in the Sea State component of CCI+.

Product Generation and Intercomparison

Space Agencies all fulfil the requirements for GDR generation – although there is now a distinct break with the GDR concepts in the Sentinel-6 mission.. The multi-mission products provided by CNES /AVISO, covering more than two decades, include corrections following systematic inter-comparisons at ground track cross-overs.  Systematic inter-comparisons of lower-level data products and new geophysical corrections should also be systematically compared.  For SAR altimeters, fully-focused SAR approaches may prove more lucrative than the simple “stack” noise reduction approaches currently used.  Alternative and updated geophysical corrections are required using the best atmospheric model and other data in a systematic manner for time series.  These need to be consistent and updated based on the latest reanalysis products.

Reprocessing needs

Multi-mission reprocessing of SSH records, fully taking inter-comparison into account, are regularly undertaken by the Agencies in particular CNES, ESA and NASA. Ongoing R&D in multi-mission reprocessing continues with the ESA CCI programme, forming the basis of a developing climate service within the Copernicus Climate Change Service (C3S) that will carry through to at least 2021.

Validation and metadata

An essential aspect in the sustained success of sea level observation from space is the capacity for precise obit determination. The current capability uses the DORIS system and on GPS satellites, complemented by ground-based laser ranging stations.

Dedicated platforms (Harvest, Senetosa, Gavdos) are maintained by space agencies for calibration of the sea level signal, complemented by ground based transponders for range calibration. Validation further depends critically on the tide gauge network, an infrastructure outside the remit of the space agencies, but with a space agency role in improving the understanding needed to link the space and gauge networks. Multiple approaches to validation is required to access different parts of the uncertainty budget.

### Physical – Sea Surface Salinity

Sea surface salinity includes activities addressing quantification of the ECV on salinity of the surface ocean. This topic addresses O11 (salinity observing system), O29 (validation of salinity) and O32 (continuity of satellite SSS record).

Ocean salinity observing system (Action O11), In situ data for satellite calibration and validation, (Action O29), Satellite sea-surface salinity (Action O32).

Mission Continuity and New Missions

Three missions have pioneered the effort to observe SSS from space: ESA’s Soil Moisture and Ocean Salinity (SMOS) mission (November 2009-present), NASA/CONAE’s Aquarius/SAC-D mission (June 2011-May 2015), and NASA’s Soil Moisture Active-Passive (SMAP) mission (January 2015-present). All three missions operate at L-band frequencies (~1.4 GHz), with SMOS having a radiometer and Aquarius and SMAP both having a radiometer and an integrated radar (the latter collects surface roughness measurements needed in SSS retrieval). Measuring SSS is part of SMOS’ and Aquarius’ mission objectives, but not part of SMAP’s mission objectives. However, the similarity of Aquarius’ and SMAP’s sensors and the experience of SSS retrieval from Aquarius have enabled SSS retrieval from SMAP. ESA has approved SMOS’ operation through the end of 2019 with further extension possible upon a review in late 2018. Aquarius/SAC-D suffered a spacecraft power failure in June 2015, which ended its extended mission that was approved by NASA. SMAP is currently in its prime mission, with a two-year extended mission plan submitted to NASA for review. Currently, either ESA nor NASA have ocean salinity missions planned beyond SMOS and SMAP.

Product Evolution

The qualities of SSS products from SMOS Aquarius, and SMAP have been progressively improving as the retrieval algorithms become more mature, for example, in terms of the reduction of biases, correction of land contaminations, and mitigation of radio frequency interference (RFI) effects. Descriptions of the product evolution and latest products for SMOS SSS can be found,. Aquarius SSS have gone through four versions, with latest version (V4.0) being the latest version. The Aquarius project will release version 5.0 in October 2017 towards the end of the Aquarius Phase-F. The versions 2 and 3 of SMAP SSS products are available.

Product generation and inter-comparison

Intercomparison among SMOS, Aquarius, and SMAP SSS products have been documented in various scientific publications for various regions and phenomena, for example, for the Gulf of Mexico (e.g., Fournier et al. 2016; Fournier et al. 2016), tropical Pacific (Yin et al. 2014), global and regional statistics (Dinnat et al. 2014).

The intercomparisons identified encouraging consistency of various satellite SSS products in depicting SSS structure in different regions and phenomena while revealing discrepancies that prompt for further understand in retrieval errors.

Reprocessing Needs

The discrepancies among satellite SSS illustrated by the ongoing intercomparison illustrate the need to further improve the satellite SSS retrievals through reprocessing. The recently available SMAP data also help improve the reprocessing of SMOS and Aquarius SSS data by capitalizing on the advantages of SMAP (e.g., better land correction tables and RFI detection). Within the ESA CCI+ programme, an effort on integrating SSS datasets to a CDR will commence in 2018.

Validation, Metadata

Validation of satellite SSS are typically performed by comparing with in-situ near-surface salinity observations from the Argo array, tropical moorings, and shipboard measurements. The results of the validation have been documented through various publications as well as validation documents by the satellite projects (e.g., the validation documents in PO.DAAC for Aquarius. Important issues related to the validation include the consideration of factors that contribute to the sampling differences between satellites and in-situ platforms. These factors include (1) near-surface stratification (in the upper few meters) that can cause the difference between satellite SSS (for the upper 1-2 cm of the ocean) and in-situ measurements (e.g., typically at 5-m depth for Argo and 1-m depth for moorings), and (2) average values within satellite footprint and time windows for satellite measurements and pointwise snapshot measurements from in-situ data. These issues are discussed in a recent publication.

Capacity Building

NASA and ESA have sponsored various training workshops, webinars, and summer schools that intend to enhance the user base for satellite SSS.

### Biogeochemistry and Biology / Ecosystems

Ocean biogeochemistry addresses ECVs quantifying water-leaving radiances or reflectances (“ocean colour radiometry”, OCR) and inferencing dissolved and particulate, living and dead constituents of the seawater, such as phytoplankton pigment concentrations (chlorophyll-a in particular), coloured dissolved organic matter (CDOM), particulate organic carbon (POC) and total suspended sediments (TSM), as well as water transparency, diffuse attenuation, and photosynthetically active radiation. Among emerging products are the structure and composition of phytoplankton community and associated particles, according to its size classes or functional types, and a variety of biogeochemical and ecosystem properties, as well as the potential to detail and quantify plankton characteristics. This topic collects the CEOS agency response to O23 (in situ OCR), O24 (OC algorithms), O25 (phytoplankton biomass) and O36 (OC climate data records).

In situ ocean colour radiometry data (Action O23), Ocean colour algorithm development (Action O24), Satellite based phytoplankton biomass estimates (O25), Satellite ocean colour (O36).

Mission Continuity and New Missions

Global OCR time series originated at the end of 1990s and beginning of 2000s with NASA (SeaWiFS, MODIS), NASDA/JAXA (OCTS), ESA (MERIS), and CNES (POLDER) polar- orbiting missions. Currently, the continuity of certain global OCR measurements at <1 km spatial resolution on polar orbiting platforms (e.g., bulk phytoplankton chl a) is secured up to 2030 thanks to recurrent dedicated missions of the European Union Copernicus and the United States Suomi NPP/JPSS programmes. Copernicus provides Ocean Colour and Land Imager (OLCI) on Sentinel 3 satellites, where the first of the series was launched in Feb 2016 and follow-on B to D missions are to launch in 2018 and in development. The Suomi NPP Visible Infrared Imaging Radiometer Suite (VIIRS) was launched in 2011 and the VIIRS instrument series and product suite overseen by NOAA are approved for continuity on JPSS-1 (2017) and JPSS-2 in 2021. Many international agencies are developing new and advanced sensors to the OCR-VC, including NASA (OCI), JAXA (SGLI), ISRO (OCM-3), China (COCTS, CZI), CONAE (MSOC).  New sensors are needed to address new and emerging science questions with products that go beyond chl a, better constrain existing algorithms and ecosystem and biogeochemical models, and enable better economic and resource management in response to user and application needs.

The continuity of geostationary OCR observations is not secured.  Geostationary Ocean Colour Imager (GOCI) of KARI/KIOST is a pioneering and thus far the only geostationary OCR mission despite the evidence of improved coverage and observations of diurnal and submesoscale dynamics in coastal waters and shelf seas. GOCI is providing complimentary time resolution to the polar-orbiting sensors at 128E and an advanced GOCI-II is planned for launch in 2019 as a future geostationary platform covering the same area.

Significant research and management user interest in coastal and inland waters drives adoption of high spatial resolution sensors which are not designed specifically for OCR applications. The continuity of these instruments, such as USGS Landsat-8 OLI and EU Copernicus Sentinel 2 series, is supported through their individual programmes. They offer opportunity for high spatial resolution of ocean colour dynamics reflecting biology and coast zone geochemistry related to sedimentary loads, even if the sensors are not maximized for aquatic retrievals.

Product Evolution

OCR-VC agencies plan to support R&D to better exploit ocean surface reflectance measurements from satellite for constraining biogeochemical ECV quantities, particularly plankton and phytoplankton biomass (O25) and quantifying phytoplankton functional types that are key for research and management (e.g., HAB and fisheries) applications.

Product Generation and Intercomparison

Improvements in product generation and intercomparisons are goals of the INSITU-OCR initiative, which is working towards the development, validation and maintenance of consistent and accurate OCR data products from multiple missions, taking advantage of existing activities within CEOS agencies and across the broader community (INSITU-OCR White Paper, 2012).  One of the activities is ESA Climate Change Initiative, which has developed a 20-year time series of consolidated OCR products following GCOS and user requirements and strict algorithm evaluation and intercomparison processes. NASA ESA, EUMETSAT and NOAA in concert with other space agencies through the OCR-VC and IOCCG maintain robust efforts to intercompare national and international mission data products.  Additionally, NASA has been investing in the development of new instruments to ensure improved ocean color instruments to be used in validation and vicarious calibration of new and planned ocean color sensors, following the instrument and measurement observational requirements outlined in the INSITU-OCR white paper.  The INSITU-OCR initiative also addresses a multi-agency network consisting of sea-based measurements for use in space-based sensor system vicarious calibration (SVC), data product validation, and algorithm development. SVC, in particular, has been jointly addressed as it requires a significant investment in field infrastructure and must be used to achieve the OCR product uncertainties required for climate and user applications. .  NOAA continues to maintain the Marine Optical BuoY (MOBY) system for sustained vicarious calibration efforts in support of all existing and forthcoming OCR sensors, along with other supporting data streams.

Reprocessing needs

Multi-sensor integrated reprocessing is a high priority for ocean colour and biogeochemical products, an ongoing major challenge being the effects on multi-sensor time-series stability of the subtle differences between missions of channel suite specifications (O36). Ocean colour will continue to be an ECV addressed within ESA’s Climate Change Initiative until 2023, and NOAA’s JPSS program through 2023 and beyond.

Validation and metadata

Fiducial reference measurements for ocean colour are being advanced on inter-calibrated in situ radiometry for example via FRM4SOC. All OCR-VC agencies intend to continue the inter-calibration of in situ FRMs and the observational requirements of the INSITU-OCR white paper, including expanding engagement with national metrological (standards) institutes to work towards better quantification of errors and uncertainties associated with the in-situ instrumentation needed for satellite data product validation. As such, agency groups such as NASA Ocean Biology Processing Group (OBPG) maintains a local repository of in situ oceanographic and atmospheric data to support their regular scientific analyses. The SeaWiFS Project originally developed this system, SeaBASS, to catalog radiometric and phytoplankton pigment data used their calibration and validation activities. To facilitate the assembly of a global data set, SeaBASS was expanded with oceanographic and atmospheric data collected by participants in the SIMBIOS Program, which has aided considerably in minimizing spatial bias and maximizing data acquisition rates. Archived data include measurements of apparent and inherent optical properties, phytoplankton pigment concentrations, and other related oceanographic and atmospheric data, such as water temperature, salinity, stimulated fluorescence, and aerosol optical thickness. Data are collected using a number of different instrument packages, such as profilers, buoys, and hand-held instruments, and manufacturers on a variety of platforms, including ships and moorings. Other space agencies maintain and are further developing similar capabilities in support of their OCR missions. The OCR-VC agencies pay constant diligence to assuring the quality of the in-situ measurements and their metadata.

## Terrestrial Climate Observing System

This section addresses GCOS-200 Part II §5 (“Terrestrial Climate Observing System”).

### General

[#]

### Hydrosphere

[#]

### Cryosphere

[#]

### Biosphere

[#]

### Human Use of Natural Resources

[#]

### Potential for Latest and Sensible Heat Flux from Land to be an ECV

[#]

Acronyms

| 3D | Three Dimensional |
| --- | --- |
| BAMS | Bulletin of American Meteorological Society |
| ABI | Advanced Baseline Imagers |
| ACE-FTS | Atmospheric Chemistry Experiment-Fourier Transform Spectrometer |
| ACP | UN African, Caribbean and Pacific (ACP) partner countries |
| ADM-Aeolus | ESA Atmospheric Dynamics Mission Aeolus |
| AERONET | Aerosol Robotic Network |
| AMSR | Advanced Microwave Scanning Radiometer |
| AMSR-E | Advanced Microwave Scanning Radiometer for EOS |
| AMSR2 | Advanced Microwave Scanning Radiometer 2 for GCOM |
| AMV | Atmospheric Motion Vector |
| AOD | Aerosol Optical Depth |
| AR | IPCC Assessment Report |
| BSRN | Baseline Surface Radiation Network |
| BUFR | Binary Universal Form for the Representation of Meteorological Data |
| CAS | Chinese Academy of Sciences |
| CCI | ESA Climate Change Initiative |
| CCI | WMO Commission for Climatology |
| CCI+ | Extension to the ESA Climate Change Initiative |
| CDR | Climate Data Record |
| CEOS | Committee on Earth Observation Satellites |
| CGMS | Coordination Group for Meteorological Satellites |
| C-GTOS | Coastal-GTOS |
| CH4 | Methane |
| CMA | China Meteorological Administration |
| CMRS | Climate Monitoring, Research and Service |
| CM SAF | Satellite Application Facility on Climate Monitoring |
| CNES | Centre National D’études Spatiales |
| CNSA | China National Space Administration |
| CONAE | Comision Nacional de Actividades Espaciales |
| COP | Conference of the Parties |
| CO | Carbon Monoxide |
| CO2 | Carbon Dioxide |
| C3S | Climate Change Service |
| DARF | Direct Aerosol Radiative Forcing |
| DEM | Digital Elevation Map |
| DLR | Deutsches Zentrum für Luft- und Raumfahrt |
| DOI | Digital Object Identifier |
| DORIS | Doppler Orbitography and Radiopositioning Integrated by Satellite |
| DMSP | The Defense Meteorological Satellite Program |
| EarthCARE | Earth Cloud Aerosol and Radiation Explorer |
| ECMWF | European Centre for Medium-Range Weather Forecasts |
| ECV | Essential Climate Variable |
| EMEP | European Monitoring and Evaluation Programme |
| EO | Earth Observation |
| EOS | Earth Observing System platforms |
| EOS AQUA | Earth Observing System Aqua platform |
| ERB | Earth Radiation Budget |
| ESA | European Space Agency |
| EUMETSAT | European Organisation for the Exploitation of Meteorological Satellites |
| FAPAR | Fraction of Absorbed Photosynthetically Active Radiation |
| FCDR | Fundamental Climate Data Record |
| FMI | Finnish Meteorological Institute |
| FRP | Fire Radiative Power |
| FY3C | FengYun-3 C |
| FY3D | FengYun-3 D |
| GAIA-CLIM | Gap Analysis for Integrated Atmospheric ECV CLImate Monitoring |
| GALION | GAW Aerosol Lidar Observation Network |
| GAS | Greenhouse-gases Absorption Spectrometer |
| GAW | Global Atmosphere Watch |
| GCI | GEOSS Common Infrastructure |
| GCOM | Global Change Observation Mission (JAXA) |
| GCOS | Global Climate Observing System |
| GCOS IP | GCOS Implementation Plan |
| GDR | Geophysical Data Record |
| GEO | Group on Earth Observations |
| GEO | Geosynchronous Earth Orbit |
| GEOSS | Global Earth Observation System of Systems |
| GEWEX | Global Energy and Water Cycle Experiment |
| GF5 | Gao Fen 5 |
| GFCS | Global Framework for Climate Services |
| GHG | Greenhouse Gas |
| GHRSST | Global High Resolution Sea Surface Temperature Project |
| GMI | GPM’s Microwave Imager |
| GNSS | Global Navigation Satellite System |
| GODAE | Global Ocean Data Assimilation Experiment |
| GOES | Geostationary Operational Environmental Satellite |
| GO-SHIP | Global Ocean Ship-based Hydrographic Investigations Program |
| GPCP | Global Precipitation Climatology Project |
| GPM | Global Precipitation Measurement mission |
| GPS | Global Positioning System |
| GRACE-FO | Gravity Recovery and Climate Experiment Follow-on |
| GRUAN | GCOS Reference Upper-Air Network |
| GSICS | WMO Global Space-based Inter-Calibration System |
| GSN | GCOS Surface Network |
| GTN-G | Global Terrestrial Network for Glaciers |
| GTN-H | Global Terrestrial Network for Hydrology |
| GTN-L | Global Terrestrial Network for Lake Level/Area |
| GTN-P | Global Terrestrial Network for Permafrost |
| GTN-R | Global Terrestrial Network for River Discharge |
| GTOS | Global Terrestrial Observing System |
| GUAN | GCOS Upper Air Network |
| HCHO | Formaldehyde |
| HY | Hai Yang |
| ICDR | Interim Climate Data Record |
| ICSU | International Council for Science |
| ILSTE | International Land Surface Temperature and Emissivity Working Group |
| INPE | Instituto Nacional de Pesquisas Espaciais |
| IOC | Intergovernmental Oceanographic Commission |
| IPCC | Intergovernmental Panel on Climate Change |
| IPMA | Instituto Português do Mar e da Atmosfera |
| IPWG | International Precipitation Working Group |
| iQUAM | In-situ SST Quality Monitor |
| IR | Infrared |
| ISS | International Space Station |
| JAXA | Japan Aerospace Exploration Agency |
| JPSS | Joint Polar Satellite System |
| JRC | Joint Research Centre |
| KMA | Korea Meteorological Administration |
| LAI | Leaf Area Index |
| LEO | Low Earth orbit |
| LIDAR | Light Detection and Ranging |
| LS | Lower Stratisphere |
| LULUCF | Land Use, Land-Use Change and Forestry |
| MAESTRO | Measurements of Aerosol Extinction in the Stratosphere and Troposphere Retrieved by Occultation |
| MOE | Ministry of the Environment |
| MOST | Ministry of Science and Technology |
| MSG | Meteosat Second Generation |
| MTG | Meteosat Third Generation |
| MW | Microwave |
| N2O | Nitrous Oxide |
| NESDIS | National Environmental Satellite, Data, and Information Service |
| NMI | National Metrology Institute |
| NIES | National Institute for Environmental Studies (NIES) |
| NIR | Near-Infrared |
| NASA | National Aeronautics and Space Administration |
| NCEO | National Centre for Earth Observation |
| NDAAC | Network for the Detection of Atmospheric Composition Change |
| NH3 | Ammonia |
| NO2 | Nitrogen Dioxide |
| NOAA | National Oceanic and Atmospheric Administration |
| NPP | National Polar-orbiting Partnership |
| NSIDC | National Snow and Ice Data Center |
| NWP | Numerical Weather Prediction |
| O3 | Ozone |
| OC | Ocean Colour |
| OLR | Outgoing Longwave Radiation |
| OSIRIS | Optical Spectrograph and InfraRed Imager System |
| OSSE | Observing System Simulation Experiments |
| pCO2 | Partial Pressure of Carbon Dioxide |
| PO.DAAC | Physical Oceanography Distributed Active Archive Center |
| PPB | Parts Per Billion |
| PPBV | Parts Per Billion By Volume |
| PPM | Parts Per Million |
| REAS | Regional Emission inventory in Asia |
| RCC | WMO Regional Climate Centre |
| REDD+ | Reducing emissions from deforestation and forest degradation in developing countries, and the role of conservation, sustainable management of forests, and enhancement of forest carbon stocks in developing countries (UNFCCC) |
| RFI | Radio Frequency Interference |
| RO | Radio Occultation |
| SAC-D | Satélite de Aplicaciones Científicas-D |
| SAGE | Stratospheric Aerosol Gas Experiment |
| SAR | Synthetic Aperture Radar |
| SBSTA | Subsidiary Body for Scientific and Technological Advice |
| SCOPE-CM | Sustained and Coordinated Processing of Environmental satellite data for Climate Monitoring |
| SDG | Sustainable Development Goals |
| SEVIRI | MSG Spinning Enhanced Visible and Infrared Imager |
| SF6 | Sulfur Hexafluoride |
| SI | Systeme International ; International System of Units |
| SIF | Solar-induced Fluorescence |
| SMAP | Soil Moisture Active-Passive |
| SMMR | Nimbus-7 Scanning Multichannel Microwave Radiometer |
| SMOS | Soil Moisture and Ocean Salinity |
| SO2 | Sulfur Dioxide |
| SSH | Sea Surface Height |
| SSMI | Special Sensor Microwave Image |
| SSMIS | Special Sensor Microwave Imager/Sounder |
| SSS | Sea-surface Salinity |
| SST | Sea Surface Temperature |
| SW | Short Wave |
| SWIR | Short Wave Infrared |
| SWOT | Surface Water & Ocean Topography mission |
| SYNOP | Surface Synoptic Observations |
| TBC | To Be Confirmed |
| TCCON | Total Carbon Column Observing Network |
| TCDR | Thematic Climate Data Record |
| TIR | Thermal infrared |
| TOA | Top-of-Atmosphere |
| TOPEX | Ocean Topography Experiment, |
| TRMM | Tropical Rainfall Measuring Mission |
| USGS | United States Geological Survey |
| UT | Upper Troposphere |
| Vlab | WMO-CGMS Virtual Laboratoy for Education and Training in Satellite Meteorology |
| UN | United Nations |
| UK | United Kingdom |
| UNEP | United Nations Environment Programme |
| UNESCO | United Nations Educational, Scientific, and Cultural Organization |
| UNFCCC | United Nations Framework Convention on Climate Change |
| USA | United States of America |
| USGS | United States Geological Survey |
| UT | Upper Troposphere |
| VIRR | Visible and Infrared Radiometer |
| VIS | Visible Imaging Spectrometer |
| WGClimate | The Joint CEOS/CGMS Working Group on Climate |
| WGCV | CEOS Working Group on Calibration & Validation |
| WGISS | CEOS Working Group on Information Systems and Services |
| WMO | World Meteorological Organization |
| XCTD | Expendable Conductivity Temperature Depth |
| XBT | Expendable Bathythermograph |

Mapping From GCOS IP to Space Agency Response

This Annex provides a mapping from the GCOS IP to the content herein, and comprises (1) document mapping from GCOS Implementation Plan (GCOS 200) sections to sections of this document, and (2) an Actions mapping denoting all relevant GCOS Actions responded herein.

| GCOS IP |  |  |  | Space Agency Response |  |
| --- | --- | --- | --- | --- | --- |
| Adaptation (§3.1) | G1 | Guidance and best practice for adaptation observations | - | §3.1.1 | Adaptation |
|  | G2 | Specification of high-resolution data | - | §3.1.1 | Adaptation |
| Climate Indicators (§3.3) | G3 | Development of indicators of climate change | - | §3.1.2 | Climate Indicators |
|  | G4 | Indicators for Adaptation and Risk | - | §3.1.2 | Climate Indicators |
| Sendai Framework (§4.4) | G5 | Identification of global climate observation synergies with other Multilateral Environmental Agreements (MEA) |  |  |  |
| GCOS Cooperating Mechanism (§6.1) | G6 | Assisting Developing Countries to maintain or renovate climate observation systems and to improve climate observations networks |  |  |  |
| National Coordination (§6.2) | G7 | GCOS Coordinator |  |  |  |
| Regional Activities (§6.3) | G8 | Regional Workshops |  |  |  |
| Information & Communication (§6.4) | G9 | Communications strategy |  |  |  |
| Requirements for Climate Observations (§2.1) | G10 | Maintain ECV Requirements |  |  |  |
| Planning, Review & Oversight (§2.2) | G11 | Review of CDR availability | - | §4.1.1 | Planning, Review & Oversight |
|  | G12 | Gap-analysis of CDR | - | §4.1.1 | Planning, Review & Oversight |
|  | G13 | Review of ECV observation networks |  |  |  |
|  | G14 | Maintain and Improve Coordination |  |  |  |
| Data Management, Stewardship & Access (§2.3) | G15 | Open Data Policies | - | §4.1.2 | Data Management, Stewardship & Access |
|  | G16 | Metadata |  |  |  |
|  | G17 | Support to National Data Centres |  |  |  |
|  | G18 | Long term accessibility of data |  |  |  |
|  | G19 | Data access and discoverability |  |  |  |
|  | G20 | Use of Digital Object Identifier for data records | - | §4.1.2 | Data Management, Stewardship & Access |
|  | G21 | Collaboration with WMO CCl on climate data management |  |  |  |
| Production of Integrated ECV Products (§2.4) | G22 | Implementation of new production streams in global reanalysis | - |  |  |
|  | G23 | Develop coupled reanalysis |  |  |  |
|  | G24 | Improve capability of long-range reanalysis |  |  |  |
|  | G25 | Implementation of regional reanalysis |  |  |  |
|  | G26 | Preservation of early satellite data | - | §4.1.3 | Production of Integrated ECV Products |
|  | G27 | Recovery of instrumental climate data |  |  |  |
|  | G28 | Register of data recovery activities |  |  |  |
|  | G29 | Scanned records |  |  |  |
|  | G30 | Historical data records sharing |  |  |  |
| Ancillary & Additional Observations (§2.5) | G31 | Improve Gravimetric Measurements form Space | - | §4.1.4 | Ancillary & Additional Observations |
|  | G32 | Improved Bathymetry | - | §4.1.4 | Ancillary & Additional Observations |
| Atmospheric Domain - Near-Surface Variables (§3.1) | A1 | Historical GSN availability |  |  |  |
|  | A2 | Land database |  |  |  |
|  | A3 | International exchange of SYNOP and CLIMAT reports |  |  |  |
|  | A4 | Surface Observing stations transition to automatic |  |  |  |
|  | A5 | Transition to BUFR |  |  |  |
|  | A6 | Air temperature measurements |  |  |  |
|  | A7 | Atmospheric pressure sensors on drifting buoy |  |  |  |
|  | A8 | Provide precipitation data to the Global Precipitation Climatology Centre |  |  |  |
|  | A9 | Submit Water Vapour data |  |  |  |
|  | A10 | National sunshine records into Data Centres |  |  |  |
|  | A11 | Operation of the BSRN | - | §4.2.3 | Satellite Data Calibration, Validation and Reprocessing |
|  | A12 | Surface Radiation Data into WRDC |  |  |  |
| Atmospheric Domain - Upper-Air (§3.2) | A13 | Implement vision for future of GUAN operation |  |  |  |
|  | A14 | Evaluation of benefits for GUAN |  |  |  |
|  | A15 | Implementation of GRUAN | - | §4.2.3 | Satellite Data Calibration, Validation and Reprocessing |
|  | A16 | Implementation of satellite calibration missions | - | §4.2.3 | Satellite Data Calibration, Validation and Reprocessing |
|  | A17 | Retain original measured values for radiosonde data |  |  |  |
|  | A18 | Hyperspectral radiances reprocessing | - | §4.2.3 | Satellite Data Calibration, Validation and Reprocessing |
|  | A19 | AMV reprocessing | - | §4.2.1 | Atmospheric Dynamics, Energy and Water Cycle Components |
|  | A20 | Increase the coverage of aircraft observations |  |  |  |
|  | A21 | Implementation of space-based wind profiling system | - | §4.2.1 | Atmospheric Dynamics, Energy and Water Cycle Components |
|  | A22 | Develop a repository of water vapour CDRs | - | §4.2.2 | Atmospheric Composition Including Greenhouse Gases |
|  | A23 | Measure of water vapour in the UT/LS |  |  |  |
|  | A24 | Implementation of archive for radar reflectivities |  |  |  |
|  | A25 | Continuity of global satellite precipitation products | - | §4.2.1 | Atmospheric Dynamics, Energy and Water Cycle Components |
|  | A26 | Development of methodology for consolidated precipitation estimates | - | §4.2.1 | Atmospheric Dynamics, Energy and Water Cycle Components |
|  | A27 | Dedicated satellite ERB mission | - | §4.2.1 | Atmospheric Dynamics, Energy and Water Cycle Components |
|  | A28 | In-situ Profile and Radiation |  |  |  |
|  | A29 | Lightning | - | §4.2.1 | Atmospheric Dynamics, Energy and Water Cycle Components |
| Atmospheric Domain -Composition (§3.3) | A30 | Water vapour and ozone measurement in UT/LS and upper stratosphere | - | §4.2.2 | Atmospheric Composition Including Greenhouse Gases |
|  | A31 | Validation of satellite remote sensing | - | §4.2.3 | Satellite Data Calibration, Validation and Reprocessing |
|  | A32 | FDCRs and CDRs for GHG and aerosols ECVs | - | §4.2.2 | Atmospheric Composition Including Greenhouse Gases |
|  | A33 | Maintain WMO GAW CO2 and CH4 monitoring networks |  |  |  |
|  | A34 | Requirements for in-situ column composition measurements |  |  |  |
|  | A35 | Space-based measurements of C02 and CH4 implementation | - | §4.2.2 | Atmospheric Composition Including Greenhouse Gases |
|  | A36 | N2O, halocarbon and SF6 networks/measurements | - | §4.2.3 | Satellite Data Calibration, Validation and Reprocessing |
|  | A37 | Ozone networks coverage | - | §4.2.2 | Atmospheric Composition Including Greenhouse Gases |
|  | A38 | Submission and dissemination of ozone data |  |  |  |
|  | A39 | Monitoring of aerosols properties | - | §4.2.2 §4.2.3 | Atmospheric Composition Including Greenhouse Gases Satellite Data Calibration, Validation and Reprocessing |
|  | A40 | Continuity of products of precursors of ozone and secondary aerosols | - | §4.2.2 | Atmospheric Composition Including Greenhouse Gases |
| Oceanic Domain - Overview (§4.1) | O1 | Coordination of enhanced shelf and coastal Observations for climate |  |  |  |
|  | O2 | Integration and Data Access |  |  |  |
|  | O3 | Data Quality |  |  |  |
|  | O4 | Development of climatologies and reanalysis products |  |  |  |
|  | O5 | Sustained support for ocean observations |  |  |  |
|  | O6 | Technology development |  |  |  |
|  | O7 | Observing System development and evaluation |  |  |  |
| Oceanic Domain - Physical (§4.2) | O8 | Satellite SST Product Development | - | §4.3.1 | (Oceanic) Physical - Sea Surface Thermodynamic State |
|  | O9 | Upper ocean temperature observing system | - | §4.3.3 | (Oceanic) Physical - Sea Level |
|  | O10 | Full depth temperature observing system |  |  |  |
|  | O11 | Ocean salinity observing system | - | §4.3.4 | (Oceanic) Sea Surface Salinity |
|  | O12 | Ocean current gridded products | - | §4.3.2 | (Oceanic) Physical - Sea Surface Dynamic State |
|  | O13 | Sea level observations | - | §4.3.3 | (Oceanic) Physical - Sea Level |
|  | O14 | Contributing to Sea State climatologies |  |  |  |
|  | O15 | In situ sea ice observations |  |  |  |
|  | O16 | Ocean Surface Stress observations. | - | §4.3.2 | (Oceanic) Physical - Sea Surface Dynamic State |
|  | O17 | Ocean surface heat flux observing system | - | §4.3.1 | (Oceanic) Physical - Sea Surface Thermodynamic State |
| Oceanic Domain - Biogeochemistry (§4.3) | O18 | Surface pCO2 moorings |  |  |  |
|  | O19 | Building multidisciplinary timeseries. |  |  |  |
|  | O20 | Nutrient observation standards and best practices. |  |  |  |
|  | O21 | Sustaining tracer observations. |  |  |  |
|  | O22 | Develop sustained N2O observations |  |  |  |
|  | O23 | In situ ocean colour radiometry data | - | §4.3.5 | (Oceanic) Biochemistry & Biology / Ecosystems |
|  | O24 | Ocean Colour algorithm development. | - | §4.3.5 | (Oceanic) Biochemistry & Biology / Ecosystems |
| Oceanic Domain - Biology / Ecosystems (§4.4) | O25 | Satellite based phytoplankton biomass estimates | - | §4.3.5 | (Oceanic) Biochemistry & Biology / Ecosystems |
|  | O26 | Expand Continuous Plankton Recorder and Supporting observations . |  |  |  |
|  | O27 | Strengthened network of Coral Reef observation sites. |  |  |  |
|  | O28 | Global networks of observation sites for Mangroves, Seagrasses, Macroalgae. |  |  |  |
| Oceanic Domain - Key Elements of the Sustained Ocean Observing System for Climate (§4.5) | O29 | In situ data for satellite calibration and validation. | - | §4.3.1 §4.3.2 §4.3.4 | (Oceanic) Physical - Sea Surface Thermodynamic State (Oceanic) Physical - Sea Surface Dynamic State (Oceanic) Sea Surface Salinity |
|  | O30 | Satellite SST by passive microwave | - |  |  |
|  | O31 | Satellite SSH | - | §4.3.1 §4.3.3 | (Oceanic) Physical - Sea Surface Thermodynamic State (Oceanic) Physical - Sea Level |
|  | O32 | Satellite SSS | - | §4.3.4 | (Oceanic) Sea Surface Salinity |
|  | O33 | Satellite Sea State | - | §4.3.2 | (Oceanic) Physical - Sea Surface Dynamic State |
|  | O34 | Satellite Ocean Surface Stress | - | §4.3.2 | (Oceanic) Physical - Sea Surface Dynamic State |
|  | O35 | Satellite Sea Ice | - | §4.3.1 | (Oceanic) Physical - Sea Surface Thermodynamic State |
|  | O36 | Satellite Ocean Colour | - | §4.3.5 | (Oceanic) Biochemistry & Biology / Ecosystems |
|  | O37 | Argo Array |  |  |  |
|  | O38 | Development of a BioArgo Array |  |  |  |
|  | O39 | Development of a Deep Argo Array |  |  |  |
|  | O40 | GO-SHIP |  |  |  |
|  | O41 | Develop fixed point time series |  |  |  |
|  | O42 | Maintain the Tropical Moored Buoy system |  |  |  |
|  | O43 | Develop time-series based biogeochemical data |  |  |  |
|  | O44 | Meteorological Moorings. |  |  |  |
|  | O45 | Wave Measurements on moorings |  |  |  |
|  | O46 | Observations of Sea Ice from buoys and visual survey |  |  |  |
|  | O47 | Sustain drifter array |  |  |  |
|  | O48 | Underway observations from Research and Servicing Vessels |  |  |  |
|  | O49 | Improve measurements from VOS |  |  |  |
|  | O50 | Improve measurements of Underway thermosalinograph data |  |  |  |
|  | O51 | Sustain Ship-of-Opportunity XBT/XCTD |  |  |  |
|  | O52 | Coordination of underway pCO2 observations and agreed best practices |  |  |  |
|  | O53 | Underway biogeochemistry observations . |  |  |  |
|  | O54 | Continuous Plankton Recorder Surveys |  |  |  |
|  | O55 | Maintain tide gauges |  |  |  |
|  | O56 | Developing a global glider observing system |  |  |  |
|  | O57 | Developing a global animal tagging observing system |  |  |  |
| Terrestrial Domain - General (§5.2) | T1 | Improve coordination of terrestrial observations | - | §3.4.1 | [#] |
|  | T2 | Develop joint plans for coastal zones | - | §3.4.1 | [#] |
|  | T3 | Terrestrial monitoring sites | - | §3.4.1 | [#] |
|  | T4 | Review of monitoring guidance | - | §3.4.1 | [#] |
|  | T5 | Develop metadata | - | §3.4.1 | [#] |
|  | T6 | Identify capacity development needs | - | §3.4.1 | [#] |
| Terrestrial Domain - Hydrosphere (§5.3) | T7 | Exchange of hydrological data |  |  |  |
|  | T8 | Lakes and Reservoirs: compare satellite and in situ observations | - | §3.4.2 | [#] |
|  | T9 | Submit historical and monthly lake level data |  |  |  |
|  | T10 | Establish sustained production and improvement for the Lake ECV products | - | §3.4.2 | [#] |
|  | T11 | Confirm GTN-R sites |  |  |  |
|  | T12 | National needs for river gauges |  |  |  |
|  | T13 | Establish full scale Global Groundwater Monitoring Information System (GGMS) |  |  |  |
|  | T14 | Operational Groundwater monitoring from gravity measurements | - | §3.4.2 | [#] |
|  | T15 | Satellite soil moisture data records | - | §3.4.2 | [#] |
|  | T16 | Multi-satellite soil moisture data services | - | §3.4.2 | [#] |
|  | T17 | International soil moisture network |  |  |  |
|  | T18 | Regional high-resolution soil moisture data record | - | §3.4.2 | [#] |
| Terrestrial Domain - Cryosphere (§5.4) | T19 | Maintain and extend the in-situ mass balance network |  |  |  |
|  | T20 | Improve the funding situation for international glacier data centres |  |  |  |
|  | T21 | Encourage and enforce research projects to make their ECV-relevant observations available through dedicated international data centres | - | §3.4.3 | [#] |
|  | T22 | Global glacier inventory | - | §3.4.3 | [#] |
|  | T23 | Multi-decadal glacier inventories | - | §3.4.3 | [#] |
|  | T24 | Allocate additional resources to extend the geodetic dataset | - | §3.4.3 | [#] |
|  | T25 | Extend the glacier front variation dataset both in space and time | - | §3.4.3 | [#] |
|  | T26 | Glacier observing sites | - | §3.4.3 | [#] |
|  | T27 | Observations of glacier velocities | - | §3.4.3 | [#] |
|  | T28 | Snow-cover and snowfall observing sites |  |  |  |
|  | T29 | Integrated analyses of snow | - | §3.4.3 | [#] |
|  | T30 | Ice sheet measurements |  |  |  |
|  | T31 | Ice sheet model improvement |  |  |  |
|  | T32 | Continuity of laser, altimetry, and gravity satellite missions | - | §3.4.3 | [#] |
|  | T33 | Standards and practices for permafrost |  |  |  |
|  | T34 | Mapping of seasonal soil freeze/thaw |  |  |  |
| Terrestrial Domain - Biosphere (§5.5) | T35 | Ensure the consistency of the various radiant energy fluxes | - | §3.4.4 | [#] |
|  | T36 | Climate change indicators for adaptation |  |  |  |
|  | T37 | Quality of ground-based reference sites for FAPAR and LAI | - | §3.4.4 | [#] |
|  | T38 | Improve snow and ice albedo products | - | §3.4.4 | [#] |
|  | T39 | Improve in situ albedo measurements |  |  |  |
|  | T40 | Production of CDRs for LAI, FAPAR, and Albedo | - | §3.4.4 | [#] |
|  | T41 | Evaluate LAI, FAPAR, and Albedo | - | §3.4.4 | [#] |
|  | T42 | Land surface temperature: in situ protocols | - | §3.4.4 | [#] |
|  | T43 | Production of land surface temperature datasets | - | §3.4.4 | [#] |
|  | T44 | Reprocessing land surface temperature | - | §3.4.4 | [#] |
|  | T45 | Land surface temperature in situ network expansion | - | §3.4.4 | [#] |
|  | T46 | Land surface temperature radiometric calibration | - | §3.4.4 | [#] |
|  | T47 | Land cover experts | - | §3.4.4 | [#] |
|  | T48 | Annual land cover products | - | §3.4.4 | [#] |
|  | T49 | Land cover change | - | §3.4.4 | [#] |
|  | T50 | Land cover community consensus | - | §3.4.4 | [#] |
|  | T51 | Deforestation | - | §3.4.4 | [#] |
|  | T52 | Collaboration on above ground biomass | - | §3.4.4 | [#] |
|  | T53 | Above ground biomass validation strategies | - | §3.4.4 | [#] |
|  | T54 | Above ground biomass validation sites | - | §3.4.4 | [#] |
|  | T55 | Above ground biomass data access | - | §3.4.4 | [#] |
|  | T56 | Above ground biomass: forest inventories | - | §3.4.4 | [#] |
|  | T57 | Soil carbon: carbon mappin |  |  |  |
|  | T58 | Soil carbon change |  |  |  |
|  | T59 | Soil carbon – histosols |  |  |  |
|  | T60 | Historic fire data | - | §3.4.4 | [#] |
|  | T61 | Operational global burned area and FRP | - | §3.4.4 | [#] |
|  | T62 | Fire maps | - | §3.4.4 | [#] |
|  | T63 | Fire validation | - | §3.4.4 | [#] |
|  | T64 | Fire disturbance model development | - | §3.4.4 | [#] |
| Terrestrial Domain - Human Use of Natural Resources (§5.6) | T65 | Anthropogenic water use |  |  |  |
|  | T66 | Pilot projects – anthropogenic water use |  |  |  |
|  | T67 | Improved global estimates of anthropogenic GHG emissions |  |  |  |
|  | T68 | Use of satellites for LULUCF emissions/removals |  |  |  |
|  | T69 | Research on the land sink |  |  |  |
|  | T70 | Use of inverse modeling techniques to support emission inventories |  |  |  |
|  | T71 | Prepare for a carbon monitoring system | - | §3.4.5 | [#] |
| Terrestrial Domain - Potential (§5.7) | T72 | Prepare for a latent & sensible heat flux ECV | - | §3.4.5 | [#] |

Contributors

This publication was written by a team led by Pascal Lecomte.

Writing Team

Lead

Pascal Lecomte, ESA

Members

Stephan Bojinski, WMO

John Dwyer, USGS

Robert Husband, EUMETSAT

Christopher Merchant, University of Reading (UK)

Ochiai Osamu, JAXA

Ed Pechorro, ESA

Jörg Schulz, EUMETSAT

Wenying Su, NASA

Contributors

Hans Bonekamp, EUMETSAT

Paula Bontempi, NASA

Régis Borde, EUMETSAT

Jan Cermak, Karlsruhe Institute of Technology, Germany

Emilio Chuvieco, University of Alcala, Spain

Carole Deniel, CNES

Chris Derksen, Environment Canada

Paul DiGiacomo, NOAA

Craig Donlon, ESA

Darren Ghent, NCEO, University of Leicester, UK

Mitch Goldberg, NOAA

Steve Goodman, NOAA

Frank-Michael Göttsche, Karlsruhe Institute of Technology, Germany

Rainer Hollmann, EUMETSAT CM SAF

Ken Holmlund, EUMETSAT

Glynn Hulley, NASA Jet Propulsion Laboratory

Chris Justice, University of Maryland, USA

Andreas Kaab, University of Oslo, Norway

Misako Kachi, JAXA

Jeffrey R. Key, NOAA/NESDIS

Ewa Kwiatkowska, EUMETSAT

Eric Leuliette, NOAA

Kari Luojus, FMI

Milton Kampel, INPE

Hiroshi Murakami, JAXA

Thomas Nagler, ENVEO, Austria

Jun Park, KMA

Frank Paul, University of Zurich, Switzerland

Simon Pinnock, ESA

Stephen Plummer, ESA

Jeff Privette, NOAA

Bruce Raup, NSIDC, USA

Dave Robinson, Rutgers University, USA

Pierre Tabary, Meteo France

Kevin Tansey, University of Leicester, UK

Oksana Tarasova, WMO

Isabel Trigo, IPMA, Portugal

Daniel Vila, CNES

Albrecht von Bargen, DLR

Martin Wooster, Kings College, University of London, UK

Claus Zehner, ESA

© The Joint CEOS/CGMS Working Group on Climate (WGClimate), 2017.

For more information contact: wgclimate@climatemonitoring.info

www.climatemonitoring.info

The right of publication in print, electronic and any other form and in any language is reserved by WMO. Short

extracts from WMO publications may be reproduced without authorization, provided that the complete source

is clearly indicated.

GCOS, 2006: Systematic Observation Requirements for Satellite-based Products for Climate: Supplemental details to the satellite-based component of the “Implementation Plan for the Global Observing System for Climate in Support of the UNFCCC”. GCOS-107, WMO, Geneva, September 2006.

GCOS, 2016: The Global Observing System for Climate: Implementation Needs. GCOS-200, WMO, Geneva, October 2016

GCOS, 2011: Systematic Observation Requirements for Satellite-based Products for Climate: Supplemental details to the satellite-based component of the ”Implementation Plan for the Global Observing System for Climate in Support of the UNFCCC (2010 Update)”. GCOS-154, WMO, Geneva, December 2011.

Bojinski et al. (2014): The concept of Essential Climate Variables in Support of Climate Research, Applications, and Policy https://doi.org/10.1175/BAMS-D-13-00047.1

GCOS-82, 2003: The Second Report on the Adequacy of the Global Observing Systems for Climate in Support of the UNFCCC, 74 pp, available at: http://www.wmo.int/pages/prog/gcos/Publications/gcos-82_2AR.pdf

GCOS-154, 2011: Systematic Observation Requirements for Satellite-based Products for Climate: Supplemental details to the satellite-based component of the ”Implementation Plan for the Global Observing System for Climate in Support of the UNFCCC (2010 Update)”. WMO, Geneva, December 2011.

WGClimate, 2016: ECV Inventory: Questionnaire Guide v2.4.

Dowell, M., P. Lecomte, R. Husband, J. Schulz, T. Mohr, Y. Tahara, R. Eckman, E. Lindstrom, C. Wooldridge, S. Hilding, J. Bates, B. Ryan, J. Lafeuille, and S. Bojinski, 2013: Strategy Towards an Architecture for Climate Monitoring from Space. Pp. 39, available at: www.ceos.org; www.wmo.int/sat; http://www.cgms-info.org/ GCOS, 2015: Status of the Global Observing System for Climate, GCOS-195, WMO, Geneva.

WMO Workshop on Operational Space-based Weather and Climate Extremes Monitoring Geneva, Switzerland 15-17 February 2017 Meeting Report.

M. Dowell, P. Lecomte, R. Husband, J. Schulz, T. Mohr, Y. Tahara, R. Eckman, E. Lindstrom, C. Wooldridge, S. Hilding, J.Bates, B. Ryan, J. Lafeuille, and S. Bojinski, 2013: Strategy Towards an Architecture for Climate Monitoring from Space. Pp. 39, available at: www.ceos.org; www.wmo.int/sat; http://www.cgms-info.org/ GCOS, 2015: Status of the Global Observing System for Climate, GCOS-195, WMO, Geneva.

WMO No. 1162. https://library.wmo.int/pmb_ged/wmo_1162_en.pdf

CEOS EO Handbook for COP-21 §5 p42

https://library.wmo.int/opac/doc_num.php?explnum_id=3418

M. Dowell, P. Lecomte, R. Husband, J. Schulz, T. Mohr, Y. Tahara, R. Eckman, E. Lindstrom, C. Wooldridge, S. Hilding, J.Bates, B. Ryan, J. Lafeuille, and S. Bojinski, 2013: Strategy Towards an Architecture for Climate Monitoring from Space. Pp. 39. Online : http://www.wmo.int/pages/prog/sat/documents/ARCH_strategy-climate-architecture-space.pdf

ESA Activities Supporting Sustainable Development, Catalogue. 2016.

L’Ecuyer, T.S., H.K. Beaudoing, M. Rodell, W. Olson, B. Lin, S. Kato, C.A. Clayson, E. Wood, J. Sheffield, R. Adler, G. Huffman, M. Bosilovich, G. Gu, F. Robertson, P.R. Houser, D. Chambers, J.S. Famiglietti, E. Fetzer, W.T. Liu, X. Gao, C.A. Schlosser, E. Clark, D.P. Lettenmaier, and K. Hilburn, 2015: The Observed State of the Energy Budget in the Early Twenty-First Century. J. Climate, 28, 8319–8346, https://doi.org/10.1175/JCLI-D-14-00556.1

https://tu-dresden.de/bu/umwelt/geo/ipg/gef/forschung/projekte/slbc_cci/slbc_cci/

GCOS-154, 2011: Systematic Observation Requirements for Satellite-based Products for Climate: Supplemental details to the satellite-based component of the ”Implementation Plan for the Global Observing System for Climate in Support of the UNFCCC (2010 Update)”. WMO, Geneva, December 2011.

CCI Toolbox. https://cci-tools.github.io/ ; http://ect-core.readthedocs.io/en/latest/ .

Poli, P., D. P. Dee, R. Saunders, V. O. John, P. Rayer, J. Schulz, K. Holmlund, D. Coppens, D. Klaes, J. E. Johnson, A. E. Esfandiari, I. V. Gerasimov, E. B. Zamkoff, A. F. Al-Jazrawi, D. Santek, M. Albani, P. Brunel, K. Fennig, M. Schroeder, S. Kobayashi, D. Oertel, W. Doehler, D. Spaenkuch, S. Bojinski (2017), Recent advances in satellite data rescue, Bull. Amer. Meteor. Soc, doi:10.1175/BAMS-D-15-00194.1. http://dx.doi.org/10.1175/BAMS-D-15-00194.1.

Buizza, R. et al., 2017:  The EU-FP7 ERA-CLIM2 project contribution to advancing science and production of Earth-system climate reanalyses. Bulletin Meteor. Amer. Soc., submitted.

A16, A18, A19, A21, A25, A26, A27, A29, A30, A32, A35, A39, A40

A11, A15, A22, A31, A36, A37

http://www.sciencedirect.com/science/article/pii/S0034425717303826

https://www.star.nesdis.noaa.gov/sod/sst/iquam/v2/about.html

http://hrdds.ifremer.fr

http://www.frm4sts.org

http://www.esa.int/Our_Activities/Observing_the_Earth/SMOS

http://aquarius.nasa.gov

http://bec.icm.csic.es/

http://www.catds.fr/

https://podaac.jpl.nasa.gov/aquarius

https://podaac.jpl.nasa.gov/datasetlist?ids=Measurement:Platform&values=Salinity%2FDensity:SMAP&view=list

http://onlinelibrary.wiley.com/doi/10.1002/2016GL070821/full

http://www.sciencedirect.com/science/article/pii/S0034425716300773

http://onlinelibrary.wiley.com/doi/10.1002/2014JC009960/abstract

http://ieeexplore.ieee.org/document/6929701/

https://podaac.jpl.nasa.gov/aquarius

Boutin et al. http://journals.ametsoc.org/doi/abs/10.1175/BAMS-D-15-00032.1

https://coastwatch.noaa.gov/

https://frm4soc.org

https://oceancolor.gfsc.nasa.gov/