CEOS WGCV 23rd Plenary Meeting

# Report on Canadian (Natural Resources Canada) Activities

# H. Peter White, Canada Centre for Remote Sensing

7-10 March, 2005

Report Introduction

In this report we talk on some activities being pursued by or in conjunction with Natural Resources Canada, related to calibration and validation. Past reports covering previous material are not be reviewed in this report.

Present Activities

- 1-km Resolution Products

  - LAI and Clumping Index (Canada-wide)
NRCan Contacts: Wenjun Chen & Richard Fernandes
In support of climate, weather, and ecological studies, Canada-wide LAI maps are routinely derived using Advanced Very High-Resolution Radiometer (AVHRR) imagery every 10 days at 1-km resolution. Similar maps are now being produced using the SPOT VEGETATION sensor. The process of calibration – validation – calibration continues with field measurements being performed at various sites across Canada (in deciduous, conifer, mixed forests, and cropland), and using Landsat TM scenes to facilitate spatial scaling to 1-km pixels. Accuracy assessment and validation continues with new site activity each year. Influences, such as terrain and BRDF continue to be evaluated.

  - Clumping (Canada-wide)
NRCan Contacts: Sylvain Leblanc & H. Peter White
In addition to LAI, vegetation canopy heterogeneity provides complementary information.  Using radiative transfer models (Five-Scale and FLAIR), national mapping of the clumping index are being derived. A methodology to incorporate a calibration – validation stage as part of the LAI mapping effort are being evaluated.

- Landsat

  - Time Dependant Calibration
NRCan Contacts: Phil Teillet
A coordinated effort on the part of several agencies has led to the specification of a definitive time-dependent radiometric calibration record for the Landsat-5 thematic mapper (TM) for its lifetime since launch in 1984. This has been implemented in the National Landsat Archive Production Systems (NLAPS) in use in North America. The specifications of this calibration include (i) anchoring of the Landsat-5 TM calibration record to the Landsat-7 ETM+ absolute radiometric calibration, (ii) new time-dependent calibration processing equations and procedures applicable to raw Landsat-5 TM data, and (iii) algorithms for recalibration computations applicable to some of the existing processed datasets in the North American context. The cross-calibration between Landsat-5 TM and Landsat-7 ETM+ was achieved using image pairs from the tandem-orbit configuration period that was programmed early in the Landsat-7 mission. The time-dependent calibration for Landsat-5 TM is based on a detailed trend analysis of data from the on-board internal calibrator. The new lifetime radiometric calibration record for Landsat-5 will facilitate the quantitative examination of a continuous, near-global dataset at 30-m scale that spans almost two decades.

  - FLUX-NET Canada
NRCan Contacts: Werner Kurz & Robert Fraser
Changes in land use and cover and natural disturbance are thought to be major controls of the dynamic sink/source balance for the immense boreal terrestrial carbon stock. Fluxnet-Canada is a national research network developed to study the influence of climate and disturbance on terrestrial carbon cycling along an east-west transect of Canadian forest and peatlands. The purpose of the present work was to create large-area land cover classifications from satellite imagery to support Fluxnet scaling and modeling studies. The methodology presented in [9] was implemented on seven Fluxnet monitoring sites across Canada. A Landsat ETM+ image covering each site was clustered to 150 classes using unsupervised K-Means classification prior to 50-cluster merging through classification by generalization (CPG). The resulting clusters were merged to 16-class landcover maps through interactive labeling using cluster bitmaps to create spatial context for the 50 clusters. The bitmaps aided the analyst when ground data was scarce or nonexistent. The products were sent to Fluxnet-Canada site managers for ground validation and the classifications were refined according to the feedback provided. The final landcover maps were used in the calculation of leaf area index (LAI) for the Landsat ETM+ scenes, thus allowing for upscaling of carbon flux measurements based on the correlation of LAI to carbon flux.

  - Calibration of National Forest Fire Impacts
NRCan Contacts: Robert Fraser
Satellite-based mapping can provide a timely and efficient means of identifying burned vegetation at continental scales for estimating greenhouse gas emissions and its effects on the terrestrial carbon budget. In this study, we used a sample of 55 Landsat Thematic Mapper (TM) scenes distributed across Canada to validate and calibrate 1998 and 1999 national-level burned areas maps produced using coarse resolution (~ 1-km) SPOT VEGETATION and NOAA AVHRR imagery. Commission and omissions errors, based on fire events > 200 ha, were found to be small in the coarse resolution maps (4% and 1%, respectively). However, the coarse resolution burned area estimates were 72 percent larger than the crown fire burned area mapped at 30 m using Landsat TM (11,039 vs. 6,403 ha average area). This bias was attributed to spatial aggregation effects in which the coarse resolution product included the tree crown fire, partial burn, and unburned fractions of a pixel. A regression calibration model (R2=0.95, p< 0.0005, RMS=3,015 ha, n=155) based on a VGT/TM double sampling approach was derived to correct for the aggregation bias and to provide Canada-wide estimates of crown fire burned area.

- CHRIS/PROBA

  - Processing of CHRIS Hyperspectral Data
NRCan Contacts: Karl Staenz
Processing for northern geological applications has been performed using the ISDAS Hyperspectral Calibration Procedure (HCP - see below). The processed data set was acquired over Nanisivik, Nunavut, Canada on July 9, 2003 using mode 5. The nadir image (GSD = 17 m) was selected for the data quality assessment. Landcover in this Arctic environment (N73°) consists mainly of exposed bedrock and tundra vegetation. Using the ISDAS-HCP, the following sensor and calibration artifacts were investigated: striping (incl. dead pixels), random noise, keystone, smile, and radiometric calibration. The at-sensor radiance data were converted to surface reflectance using look-up tables generated with MODTRAN V4.3 to calculate the reflectance on a pixel basis.

- EO-1 Hyperion

  - Forestry Applications
NRCan Contacts: David Goodenough
In cooperation with NASA, The Pacific Forestry Centre (PFC) of Natural Resources Canada are conducting the Evaluation and Validation of EO-1 for Sustainable Development of forests (EVEOSD) project. NASA’s Hyperion, ALI and Landsat-7 ETM+ data were collected over the Greater Victoria Watershed (GVWD) test site on Vancouver Island on September 10, 2001. Concurrent ground reflectance measurements were made of calibration sites. Canopy and ground cover chemistry samples were also collected. Several publications on this work can be found in the literature.

- Hyperspectral Applications

  - FLAIR
NRCan Contacts: H. Peter White
As space-borne sensors are increasingly being designed to provide off-nadir imagery (to increase the potential of target application frequency and acquisition success), the bi-directional reflectance characteristics of scenes need to be assessed for information extraction. The FLAIR Model (Four-Scale model for AnIsotropic Reflectance) is being evaluated with leaf level reflectance models to allow for the extraction of vegetation properties. In this case, the spectral imagery provides the means of separating reflectance characteristics of scene components from the architectural structure and distribution of the components.

  - HUST
NRCan Contacts: Karl Staenz & H. Peter White
In order to advance the application of hyperspectral remote sensing, a Hyperspectral User and Science Team (HUST) has been created to serve as an advisory body. This advisory team will focus on the development of appropriate products and highlight their usefulness to the user community. Application development sections includes i) Geology/Soil; ii) Forestry; iii) Agriculture; iv) Environment (including wetlands, snow/ice, and climate change) and v) Coastal/Inland Waters. 
As part of the HUST applications development plan, core test sites are being considered where application products (LAI, soil moisture, snow depth, etc.) can be evaluated on a continual bases. This will facilitate the calibration and validation of procedures and methodology development. A plan to call for candidate sites is being prepared.

  - HERO
NRCan Contacts: Karl Staenz & H. Peter White
The Hyperspectral Environment and Resource Observer (HERO) is a proposed space-borne hyper-spectral instrument. The proposed HERO mission will provide unique capabilities to discern physical and chemical properties of Earth surface features not possible using current broadband multi-spectral sensors. The HERO instrument will acquire data anywhere globally and at low cost to the end user. This program will coordinate with the HUST in the definition of mission requirements and the trade-off decisions that must be made during the development work in order to best serve the user community. The mission statement is as follows:
The Hyperspectral Environment and Resource Observer (HERO) will be a Canadian optical Earth Observation mission that will address the stewardship of natural resources for sustainable development within Canada and globally. Through targeted imaging mapping and regular monitoring of the Earth's surface, HERO will acquire and deliver high-quality hyperspectral data that will support decision-making in the management of sensitive ecosystems and valuable natural resources.

- In-situ 
NRCan Contacts: Phil Teillet
In Situ Sensor Measurement Assimilation Program (issmap)
The objective of this program is to make significant advancements in the practical use of Earth observation data by developing intelligent in situ measurement capabilities that open new pathways towards the generation of quantitative and validated geophysical and biospheric information products. This will allow for the real-time monitoring of remote environments, hazards and disasters, and natural resources.

  - CCRS In-Situ Sensorweb Prototype Demonstrations

    - Flood Hazard Monitoring: Red River Watershed, Manitoba
Satellite-based soil moisture maps and strategically deployed in situ sensorwebs can monitor soil moisture changes in space and time without ongoing field work. Measurements include: Global radiation, relative humidity and precipitation, air and soil temperature, rainfall, wind speed and direction, and soil moisture (Adcon C-Probes with sensors at multiple depths to match soil core sampling locations).

    - Groundwater Monitoring: Oak Ridges Moraine, Ontario
Deployment of a sensorweb to monitor spatial patterns in climate and evaporation.

    - Drought Severity and Vegetation Monitoring: Alberta
Deployment to detect the onset of drought and, more generally, support the prediction and assessment of rangeland productivity and crop yields. In this demonstration, each sensor node provides soil moisture, precipitation, and soil temperature information via wireless communication to a central server in real time.